| PROJECT NAME: |  |  | DRAWING NO. |  |  |  |  |  | REV. |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | QUESTIONS |  |  | ORIGINATOR |  |  | CHECKER |  |  |  |
|  |  |  |  | N/A | YES | NO | N/A | YES |  | NO |
| - Drawing Presentation |  |  |  |  |  |  |  |  |  |  |
|  | Does Drawing comply with the project CAD Standards (All lines, symbols, legends, abbreviations, text, etc. are legible)? |  |  |  |  |  |  |  |  |  |
|  | Is the Drawing reviewed for constructability? |  |  |  |  |  |  |  |  |  |
|  | Is the layout read and interpreted in conjunction with the applicable Architectural, Civil, Electrical and Mechanical Layout? |  |  |  |  |  |  |  |  |  |
|  | Are the fields in the title block consistent with the project drawing log/ index? |  |  |  |  |  |  |  |  |  |
|  | Key plan and North arrow are provided & the key plan shall have the layout area hatched. |  |  |  |  |  |  |  |  |  |
|  | Match lines or X-Y Grid are clearly defined. |  |  |  |  |  |  |  |  |  |
|  | Drawing notes are complete & agree with information on the drawings & details. |  |  |  |  |  |  |  |  |  |
|  | Are the layout dimensions (mounting height, operating height of the outlets, devices, etc.) clearly defined? |  |  |  |  |  |  |  |  |  |
|  | Are the layout legends specific and provide the details regarding the type of the cable, the detail of the IP Camera and accessories, Server, routers, switches, etc.? |  |  |  |  |  |  |  |  |  |
|  | Bar scale is shown on the drawing and correct scale is used for all details, plan/elevation/sections. Details Not to Scale are also clearly marked. |  |  |  |  |  |  |  |  |  |
|  | All interdisciplinary comments and comments from previous revisions have been resolved and incorporated. Holds and revisions are correctly marked. |  |  |  |  |  |  |  |  |  |
|  | Limits of existing and new work including future expansions, interface points, Battery limits are clearly defined |  |  |  |  |  |  |  |  |  |
|  | Check if conduit continuations between drawings have been verified and properly identified. |  |  |  |  |  |  |  |  |  |
| - Code/Standard/Project Specification |  |  |  |  |  |  |  |  |  |  |
|  | The system design shall comply with the applicable Code, International and Saudi standard, local Government Regulation and Project Specification |  |  |  |  |  |  |  |  |  |
|  | The installation of the system shall be in accordance with NFPA, EIA/TIA, NEC, NEMA, BICSI, IEC, IEEE and other applicable standards. |  |  |  |  |  |  |  |  |  |
|  | Verify the equipment and components tag/identification numbers, dimensions, locations, etc. with project standards/vendor documents, as applicable code |  |  |  |  |  |  |  |  |  |
|  | Make sure that each type of connection/installation is represented either by standard details applicable for the project or details are drawn on the layout drawing. |  |  |  |  |  |  |  |  |  |
| - Reference Information |  |  |  |  |  |  |  |  |  |  |
|  | Check to ensure general notes include reference to applicable Codes, Standards and Project Specifications/Supplier submittals. |  |  |  |  |  |  |  |  |  |
|  | Check for correctness of reference drawings. |  |  |  |  |  |  |  |  |  |
|  | List and verify any special requirements by others (Sub-Contractors, Vendors, etc.) |  |  |  |  |  |  |  |  |  |
|  | Are the layout provided the details of the wiring diagram? |  |  |  |  |  |  |  |  |  |
|  | Sections and details are correctly cross-referenced. |  |  |  |  |  |  |  |  |  |
|  | Have the layout specified the dedicated containment for the applicable system? |  |  |  |  |  |  |  |  |  |
|  | Does the layout provide the detail of the containment route (both the horizontal and vertical)? |  |  |  |  |  |  |  |  |  |
| - Design |  |  |  |  |  |  |  |  |  |  |
|  | Has the technical feasibility of the system includes the network equipment’s, components, video management software, etc. verified? |  |  |  |  |  |  |  |  |  |
|  | Does the layout provide the detail of the UTP and Fire Optic Cables (backbone)? |  |  |  |  |  |  |  |  |  |
|  | Does the layout provide the detail of the camera (type – Digital or Analog, model, etc.)? |  |  |  |  |  |  |  |  |  |
|  | Does the system define the power supply requirement for the camera? |  |  |  |  |  |  |  |  |  |
|  | Does the designing of the system per IP Based and PoE (Power Over Ethernet)? |  |  |  |  |  |  |  |  |  |
|  | Is the layout provided the details of related works such as Power, Raceways, structure cables, data network/LAN, etc.? |  |  |  |  |  |  |  |  |  |
|  | Does the layout, specify the type of network between the camera, switch and the server? |  |  |  |  |  |  |  |  |  |
|  | Is the layout provided the connection details with the infrastructure or Structured Cable System through TCP/IP network protocol? |  |  |  |  |  |  |  |  |  |
|  | Does the system, consider the network or network equipment includes PoE switch & wireless Antenna (where applicable)? |  |  |  |  |  |  |  |  |  |
|  | Does the system network optimization compatible with the Existing system network (If, applicable)? |  |  |  |  |  |  |  |  |  |
|  | Are the selection of the camera offer High Definition IP Surveillance solutions? |  |  |  |  |  |  |  |  |  |
|  | Does the selection and the location of peripheral devices, components, etc., as per the specialist recommendation and the project requirements? |  |  |  |  |  |  |  |  |  |
|  | Does the system network optimization compatible with the Existing system network (If, applicable)? |  |  |  |  |  |  |  |  |  |
|  | Is the system layout provided the details of the workstation and Video Wall location? |  |  |  |  |  |  |  |  |  |
|  | Are the system provided the connection details between IT Backbone Cabinet? |  |  |  |  |  |  |  |  |  |
|  | Does the system provide the details of the Server or Network Video Recorder? |  |  |  |  |  |  |  |  |  |
|  | Does the layout provided the details of Database Video storage capacity for the short or long term storage? |  |  |  |  |  |  |  |  |  |
|  | Have the system provided the provision or facilities to integrate with the ancillary system? |  |  |  |  |  |  |  |  |  |
|  | Does the system provided with provision to integrate with Access Control System, IP infrastructure and BMS System? |  |  |  |  |  |  |  |  |  |
| No. | Reviewer's Comments | Resolution |  |  |  |  |  |  |  |  |
| Originator's Name / Signature and Date: |  | Checker's Name / Signature and Date: |  |  |  |  |  |  |  |  |