| PROJECT NAME: |  |  | DRAWING NO. |  |  | REV. |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No. | INSPECTION ITEM |  |  | CHECKED SATISFACTORY |  |  |  |
|  |  |  |  | N/A | YES |  | NO |
|  | General Check |  |  |  |  |  |  |
|  | Capacity of platform/cradle or BMU traction motors including all accessories conforms to the approved documentation. Lifting capacity shall be in accordance to Specification and equipment approved MAS (Material Approval Sheet). |  |  |  |  |  |  |
|  | Installation conforms to the requirement of the contract, specifications, and drawings. Complete installation is accepted by the Client Representatives. |  |  |  |  |  |  |
|  | All bolts and nuts for rail system fixing are 100% torque tested. |  |  |  |  |  |  |
|  | Load testing conducted for each tiers and base anchors of BMU track railing by applying force equivalent to half of the anchor capacity. |  |  |  |  |  |  |
|  | Safe entry and exit provided for the workers for coming into and out of the cradle. |  |  |  |  |  |  |
|  | All metal parts are grounded. |  |  |  |  |  |  |
|  | Traction cables in good condition, lubricated, no visible entanglement, and excessive sagging. |  |  |  |  |  |  |
|  | Guide rail is structurally sound and properly installed including anchoring, brackets, bolting/fixing and framings. |  |  |  |  |  |  |
|  | Cradle is provided with bump protection and rollers. |  |  |  |  |  |  |
|  | Anti-fall anchorage provided in the cradle. |  |  |  |  |  |  |
|  | All electrical installation and testing conforms to the requirement of NFPA 70 and other electrical standards. |  |  |  |  |  |  |
|  | Equipment moving parts are properly lubricated. |  |  |  |  |  |  |
|  | Equipment safeties are in place and properly working such as cradle overload device, cradle trip bar, jib slew end of travel limit switch, cradle emergency retrieval hand wind, slack rope device, over-speed detector and brake, emergency stop, residual current device, cross bar slew end of travel limits, electrical phase failure detector, harness attachment points, cradle full-up detector, lanyard restraint trip bar, secondary cradle full-up over travel detector and wire rope equalizer. |  |  |  |  |  |  |
|  | Operational labelling provided within the cradle (emergency retrieving, up/down control, slewing, etc.). |  |  |  |  |  |  |
| No. | Reviewer's Comments | Resolution |  |  |  |  |  |
| Originator's Name / Signature and Date: |  | Checker's Name / Signature and Date: |  |  |  |  |  |