| PROJECT NAME: |  |  | DRAWING NO. |  |  |  | REV. |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | QUESTIONS |  | ORIGINATOR |  |  | CHECKER |  |  |  |
|  |  |  | N/A | YES | NO | N/A |  | YES | NO |
| 01 | Does this drawing use latest model and information of existing ground and grading? |  |  |  |  |  |  |  |  |
| 02 | Does this drawing show legends for all the entities given on drawing? |  |  |  |  |  |  |  |  |
| 03 | Does this drawing show references to all applicable typical drawings? |  |  |  |  |  |  |  |  |
| 04 | Does this drawing show reference to specifications? |  |  |  |  |  |  |  |  |
| 05 | Does this drawing specify unit of dimensions? |  |  |  |  |  |  |  |  |
| 06 | Does this drawing show horizontal and vertical scale of cross-section? |  |  |  |  |  |  |  |  |
| 07 | Is appropriate vertical exaggeration used in this drawing? |  |  |  |  |  |  |  |  |
| 08 | Is horizontal and vertical scale of cross-section appropriate and as per standard? |  |  |  |  |  |  |  |  |
| 09 | Does this drawing specify correct vertical datum reference? |  |  |  |  |  |  |  |  |
| 10 | Does this drawing show match line of other drawings? |  |  |  |  |  |  |  |  |
| 11 | Does this drawing show station of the cross-section/sections? |  |  |  |  |  |  |  |  |
| 12 | Does the station of the cross-section/sections in this drawing match with grading layout drawings? |  |  |  |  |  |  |  |  |
| 13 | Does this drawing show level information of existing ground, top soil removal, and grading? |  |  |  |  |  |  |  |  |
| 14 | Does this drawing show offset values of existing ground, top soil removal, and grading? |  |  |  |  |  |  |  |  |
| 15 | Is level information in data band correct with respect to station? |  |  |  |  |  |  |  |  |
| 16 | Are specified cut and fill slopes used for grading of site and road? |  |  |  |  |  |  |  |  |
| 17 | Are the grading elevations coordinated with building pad elevations and other nearby facilities? |  |  |  |  |  |  |  |  |
| 18 | Is minimum vertical clearance and slope between Finished Floor Level and Finished Pavement Level appropriate? |  |  |  |  |  |  |  |  |
| 19 | Is grading developed based on natural drainage pattern of area? |  |  |  |  |  |  |  |  |
| 20 | Are graded slopes, swales and ditches that are subject to erosion stabilized? |  |  |  |  |  |  |  |  |
| 21 | Are heights of cut and fill within the specified limits? |  |  |  |  |  |  |  |  |
| 22 | Are minimum transverse and minimum longitudinal gradients of graded surfaces in accordance with standards? |  |  |  |  |  |  |  |  |
| 23 | Are maximum transverse and maximum longitudinal gradients of graded surfaces in accordance with standards? |  |  |  |  |  |  |  |  |
| No. | Checker's Comments | Resolution |  |  |  |  |  |  |  |
| Originator's Name / Signature and Date: |  | Checker's Name / Signature and Date: |  |  |  |  |  |  |  |