| PROJECT NAME: |  |  | DRAWING NO. |  |  |  |  |  | REV. |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | QUESTIONS |  |  | ORIGINATOR |  |  | CHECKER |  |  |  |
|  |  |  |  | N/A | YES | NO | N/A | YES |  | NO |
|  | Does the drawing comply with applicable codes, standards and regulatory requirements? |  |  |  |  |  |  |  |  |  |
|  | Does the drawing comply with applicable Project Design Criteria, system or structural functional requirements, Scope Book, and Design Basis Documents considered? |  |  |  |  |  |  |  |  |  |
|  | Do the Allowable Bearing Capacity (ABC) and settlement on the drawing comply with geotechnical report? |  |  |  |  |  |  |  |  |  |
|  | Do concrete covers over reinforcement comply with Project Specifications? |  |  |  |  |  |  |  |  |  |
|  | Do the compressive strength of in-situ and precast concrete comply with Project Specifications? |  |  |  |  |  |  |  |  |  |
|  | Does the specified compressive strength of mass concrete comply with Project Specifications? |  |  |  |  |  |  |  |  |  |
|  | Does type of concrete/concrete mix comply with Project Specifications? |  |  |  |  |  |  |  |  |  |
|  | Does the grade of reinforcement steel comply with Project Specifications? |  |  |  |  |  |  |  |  |  |
|  | Do the splice lengths of rebars comply with code? |  |  |  |  |  |  |  |  |  |
|  | Does the backfill around the structure comply with Project Specifications? |  |  |  |  |  |  |  |  |  |
|  | Does quay wall validate the calculation report? |  |  |  |  |  |  |  |  |  |
|  | Is a shear key required for stability? |  |  |  |  |  |  |  |  |  |
|  | Does the eccentricity of wall comply the code? |  |  |  |  |  |  |  |  |  |
|  | Has the effect of ground water table been considered in the calculation report? |  |  |  |  |  |  |  |  |  |
|  | Has geotextile behind the wall been provided where required by design? |  |  |  |  |  |  |  |  |  |
|  | Has the stability of wall been checked for the construction stage? |  |  |  |  |  |  |  |  |  |
|  | Has the weight of precast blocks/sections been reduced for lifting pockets in stability calculations? |  |  |  |  |  |  |  |  |  |
|  | Has the effect of tidal lag been considered in the stability analysis of wall? |  |  |  |  |  |  |  |  |  |
|  | Have the precast sections/blocks been checked for maximum tensile stress for lifting stage? |  |  |  |  |  |  |  |  |  |
|  | Does the drawing specify the minimum compressive strength of precast concrete sections/blocks for lifting stage? |  |  |  |  |  |  |  |  |  |
|  | Do the fender & bollard spacing comply with calculations? |  |  |  |  |  |  |  |  |  |
|  | Does edge distance of bollard comply with code? |  |  |  |  |  |  |  |  |  |
|  | Does ladder spacing comply with code? |  |  |  |  |  |  |  |  |  |
|  | Does the length of ladder below the LAT (Lowest Astronomic Tide) comply with code? |  |  |  |  |  |  |  |  |  |
|  | Has the fender been checked for all correction factors (Temp., velocity and berthing angle)? |  |  |  |  |  |  |  |  |  |
|  | Has sufficient scour protection been provided in front of quay wall? |  |  |  |  |  |  |  |  |  |
|  | Has corrosion allowance been considered for sheet pile quay wall? |  |  |  |  |  |  |  |  |  |
|  | Has the quay wall been checked for erosion/scour from overtopping? |  |  |  |  |  |  |  |  |  |
|  | Has the quay wall been checked for high and low water levels as used in the design? |  |  |  |  |  |  |  |  |  |
|  | Has the vertical datum of quay wall been checked? |  |  |  |  |  |  |  |  |  |
|  | Has reference of General Notes drawing been provided? |  |  |  |  |  |  |  |  |  |
|  | Have references of interface drawings (MEP) been provided? |  |  |  |  |  |  |  |  |  |
|  | Has reference list of drawings been provided? |  |  |  |  |  |  |  |  |  |
|  | Has the drawing been checked for constructability? |  |  |  |  |  |  |  |  |  |
|  | Has appropriate interdisciplinary and intradepartmental coordination been done? |  |  |  |  |  |  |  |  |  |
|  | Have reviewers from other disciplines/departments evaluated items pertinent to their area and provided their comments? |  |  |  |  |  |  |  |  |  |
|  | Has the Responsible engineer resolved their comments? |  |  |  |  |  |  |  |  |  |
|  | If applicable has the drawing been stamped by a registered professional engineer? |  |  |  |  |  |  |  |  |  |
|  | Are the Drawing number and template in accordance with the project requirements? |  |  |  |  |  |  |  |  |  |
|  | If revised, have revisions been clearly identified with clouds and clear statement in revision block? |  |  |  |  |  |  |  |  |  |
|  | Have previous revision indications been removed? |  |  |  |  |  |  |  |  |  |
| No. | Checker’s Comments | Resolution |  |  |  |  |  |  |  |  |
| Originator's Name / Signature and Date: |  | Checker's Name / Signature and Date: |  |  |  |  |  |  |  |  |