| PROJECT NAME: |  |  | DRAWING NO. |  |  |  | REV. |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | QUESTIONS |  | ORIGINATOR |  |  | CHECKER |  |  |  |
|  |  |  | N/A | YES | NO | N/A |  | YES | NO |
| 1 | Does this drawing use the current building Architectural background? |  |  |  |  |  |  |  |  |
| 2 | Does this drawing show other physical features such as boundary wall, fence, drainage and hydrologic features, etc.? |  |  |  |  |  |  |  |  |
| 3 | Does this drawing show gridlines of coordinates, if a coordinate system is used? |  |  |  |  |  |  |  |  |
| 4 | Does this drawing show legends for all the entities shown on drawing? |  |  |  |  |  |  |  |  |
| 5 | Does this drawing show North Arrow that also includes Makkah direction? |  |  |  |  |  |  |  |  |
| 6 | Does this drawing show the key map of project area? |  |  |  |  |  |  |  |  |
| 7 | Does this drawing show sheet key? |  |  |  |  |  |  |  |  |
| 8 | Does this drawing showing reference to all typical applicable drawings? |  |  |  |  |  |  |  |  |
| 9 | Does this drawing mention reference specifications? |  |  |  |  |  |  |  |  |
| 10 | Does drawing specify units of dimensions? |  |  |  |  |  |  |  |  |
| 11 | Does drawing specify coordinate or baseline reference system and vertical datum? |  |  |  |  |  |  |  |  |
| 12 | Does drawing show match line of others drawings? |  |  |  |  |  |  |  |  |
| 13 | Is drawing based on latest road model? |  |  |  |  |  |  |  |  |
| 14 | Does this drawing show the centerline/baseline of road with station? |  |  |  |  |  |  |  |  |
| 15 | Are road curb types clearly shown and their limits defined? |  |  |  |  |  |  |  |  |
| 16 | Is pedestrian ramp (if any) for road crossing shown on drawing? |  |  |  |  |  |  |  |  |
| 17 | Does this drawing show a horizontal alignment which complies with the minimum radius corresponding to the design speed? |  |  |  |  |  |  |  |  |
| 18 | Does this drawing show a horizontal alignment which complies with the minimum sight distances corresponding to the design speed? |  |  |  |  |  |  |  |  |
| 19 | Does this drawing show a horizontal alignment which complies with minimum curve length? |  |  |  |  |  |  |  |  |
| 20 | Does this drawing show a horizontal alignment which complies with minimum and maximum tangent lengths? |  |  |  |  |  |  |  |  |
| 21 | Does this drawing show a horizontal alignment with correct spiral length if any? |  |  |  |  |  |  |  |  |
| 22 | Does this drawing show the correct super elevation values? |  |  |  |  |  |  |  |  |
| 23 | Does this drawing show the correct number of lanes and lane width? |  |  |  |  |  |  |  |  |
| 24 | Does this drawing show the correct taper and lane drop lengths if any? |  |  |  |  |  |  |  |  |
| 25 | Does this drawing show the correct deceleration and acceleration lengths if any? |  |  |  |  |  |  |  |  |
| 26 | Does this drawing show the correct lane widening if any? |  |  |  |  |  |  |  |  |
| 27 | Does this drawing show a vertical alignment (profile) which complies with the minimum and maximum grade corresponding to the classification of the road? |  |  |  |  |  |  |  |  |
| 28 | Does this drawing show a vertical alignment (profile) which complies with the minimum K value corresponding to the design speed? |  |  |  |  |  |  |  |  |
| 29 | Does this drawing show a vertical alignment (profile) which complies with the minimum sight distances corresponding to the design speed? |  |  |  |  |  |  |  |  |
| 30 | Does this drawing show a vertical alignment (profile) which complies with the minimum vertical curve length? |  |  |  |  |  |  |  |  |
| 31 | Does the profile on this drawing show existing ground levels, advanced grading (if any) and road design levels? |  |  |  |  |  |  |  |  |
| 32 | Does the profile on this drawing show the horizontal and vertical geometry of road? |  |  |  |  |  |  |  |  |
| 33 | Does the profile on this drawing show the station and super elevation of road? |  |  |  |  |  |  |  |  |
| 34 | Does the profile on this drawing show vertical curve data including high and low points? |  |  |  |  |  |  |  |  |
| 35 | Does the plan view drawing indicate required removals or demolitions? |  |  |  |  |  |  |  |  |
| 36 | Does the profile on this drawing show design flood level and freeboard at water crossings? |  |  |  |  |  |  |  |  |
| No. | Checker's Comments | Resolution |  |  |  |  |  |  |  |
| Originator's Name / Signature and Date: |  | Checker's Name / Signature and Date: |  |  |  |  |  |  |  |