| PROJECT NAME: |  |  | DRAWING NO. |  |  |  |  | REV. |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | QUESTIONS |  |  | ORIGINATOR |  |  | CHECKER |  |  |  |
|  |  |  |  | N/A | YES | NO | N/A |  | YES | NO |
| Preparation and Checking |  |  |  |  |  |  |  |  |  |  |
| 01 | Does the drawing comply with applicable codes, standards and regulatory requirements? |  |  |  |  |  |  |  |  |  |
| 02 | Does the drawing comply with applicable project design criteria, system or structural functional requirements, Scope Book, and Design basis documents considered? |  |  |  |  |  |  |  |  |  |
| 03 | Do the connections validate the design calculations reports? |  |  |  |  |  |  |  |  |  |
| 04 | Do the grades of structural connection bolt comply with project Specifications? |  |  |  |  |  |  |  |  |  |
| 05 | Are the type, diameters and coating i.e. galvanized, of structural bolts shown on the drawing? |  |  |  |  |  |  |  |  |  |
| 06 | Does the grade of nuts comply with project specifications? |  |  |  |  |  |  |  |  |  |
| 07 | Does the grade of connection plates (stiffener plates) comply with project Specifications? |  |  |  |  |  |  |  |  |  |
| 08 | Does the hole size of bolt in connection plates comply with code and with the assumptions made in the calculation? |  |  |  |  |  |  |  |  |  |
| 09 | Does the edge distance of bolt in connection plate (or in flange or web of rolled section) comply with code? |  |  |  |  |  |  |  |  |  |
| 10 | Does the center to center distance between bolts comply with code? |  |  |  |  |  |  |  |  |  |
| 11 | Have all types of connections been included on the drawing? |  |  |  |  |  |  |  |  |  |
| 12 | Is type of connection. i.e. snug tight, fully pre-tensioned or slip critical clearly shown on the drawing? |  |  |  |  |  |  |  |  |  |
| 13 | Has the clash between steel column and nut or washer been checked? |  |  |  |  |  |  |  |  |  |
| 14 | Have two dissimilar metals been isolated to avoid corrosion due to galvanic action? |  |  |  |  |  |  |  |  |  |
| 15 | Has correct coefficient of friction been used for friction grip (slip critical) bolts? |  |  |  |  |  |  |  |  |  |
| 16 | Does type of steel connections (shear or moment) match with design report? |  |  |  |  |  |  |  |  |  |
| 17 | Does grade of electrodes comply with code? |  |  |  |  |  |  |  |  |  |
| 18 | Has compatibility of electrodes with respect grade of steel been checked? |  |  |  |  |  |  |  |  |  |
| 19 | Has compatibility of weld thickness with Plate thickness been checked? |  |  |  |  |  |  |  |  |  |
| 20 | Does drawing include all relevant welding symbols? |  |  |  |  |  |  |  |  |  |
| 21 | Has a note for minimum weld size provided in the drawing? |  |  |  |  |  |  |  |  |  |
| Co-related Documents |  |  |  |  |  |  |  |  |  |  |
| 22 | Has reference been provided for the list of drawing? |  |  |  |  |  |  |  |  |  |
| 23 | Has reference been provided for the General Notes drawing? |  |  |  |  |  |  |  |  |  |
| 24 | Has reference been provided for the Framing Plan showing connection types (shear or moment etc )? |  |  |  |  |  |  |  |  |  |
| Coordination and Review |  |  |  |  |  |  |  |  |  |  |
| 25 | Has appropriate interdisciplinary and intradepartmental coordination been done? |  |  |  |  |  |  |  |  |  |
| 26 | Have reviewers from other disciplines/departments evaluated items pertinent to their area and provided their comments? |  |  |  |  |  |  |  |  |  |
| 27 | Has the Responsible engineer resolved their comments? |  |  |  |  |  |  |  |  |  |
| 28 | If applicable has the drawing been stamped by a registered professional engineer? |  |  |  |  |  |  |  |  |  |
| 29 | Has the drawing been checked for constructability? |  |  |  |  |  |  |  |  |  |
| Administrative |  |  |  |  |  |  |  |  |  |  |
| 30 | Is the Drawing number and template in accordance with the Project Requirements? |  |  |  |  |  |  |  |  |  |
| 31 | If revised, have revisions been clearly identified with clouds and clear statement in revision block? |  |  |  |  |  |  |  |  |  |
| 32 | Have previous revision indications been removed? |  |  |  |  |  |  |  |  |  |
| No. | Reviewer's Comments (against each SLD) | Resolution |  |  |  |  |  |  |  |  |
| Originator's Name / Signature and Date: |  | Checker's Name / Signature and Date: |  |  |  |  |  |  |  |  |