| PROJECT NAME: |  | DRAWING NO. |  |  |  | REV. |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No. | QUESTIONS | ORIGINATOR |  |  | CHECKER |  |  |
|  |  | N/A | YES | NO | N/A | YES | NO |
| Preparation and Checking |  |  |  |  |  |  |  |
| 01 | Does the drawing comply with applicable codes, standards and regulatory requirements? |  |  |  |  |  |  |
| 02 | Does the drawing comply with applicable project design criteria, system or structural functional requirements, Scope Book, and Design Basis Documents considered? |  |  |  |  |  |  |
| 03 | Have the shown structural steel framing plans/sections with indicated steel member sizes supported by the design calculations/structural analysis, which demonstrates the structural adequacy to resist the applicable design loads within the specified allowable limits, been met? |  |  |  |  |  |  |
| 04 | Have structural steel framing plans provided at each level of building? |  |  |  |  |  |  |
| 05 | Does the level on each structural framing/floor plan match with architectural Plan drawings? |  |  |  |  |  |  |
| 06 | Have the structural plans/views captured the other disciplines requirements? |  |  |  |  |  |  |
| 07 | Has clash of structural steel members been checked for MEP services/openings/recesses? |  |  |  |  |  |  |
| 08 | Has the effect of MEP openings/ducts been considered in the design? |  |  |  |  |  |  |
| 09 | Are the weights of secondary steel (additional steel required for cladding and MEP services etc.) and fireproofed steel members considered in design? |  |  |  |  |  |  |
| 10 | Was a design margin adopted when no vendor info was available or was a Not-To-Exceed load defined? |  |  |  |  |  |  |
| 11 | Have equipment & construction loads on roof/floor been considered in design? Did it follow design criteria load application? |  |  |  |  |  |  |
| 12 | Has accumulation weight of sand above the roof been considered in design? |  |  |  |  |  |  |
| 13 | Have weights of electrical and mechanical services hangings from ceiling, considered in design? |  |  |  |  |  |  |
| 14 | Has effect of temperature during construction stage (without cladding) considered in design? |  |  |  |  |  |  |
| 15 | Has additional reinforcement or additional structural steel been provided around the openings, in composite deck slab? |  |  |  |  |  |  |
| 16 | Does this drawing reflects the same material and structural configuration used in the Calculation? Is the mentioned material included in the issued project specification? |  |  |  |  |  |  |
| 17 | Do steel framing plans and sections show the type of connection (moment or shear)? |  |  |  |  |  |  |
| 18 | Do connections, clearly shown on the framing plans/sections match the connection design in the calculation report? |  |  |  |  |  |  |
| 19 | Does lateral force resisting bracing system shown on the structural steel framing plans/sections match the system assumed in the calculation? |  |  |  |  |  |  |
| 20 | Does slenderness ratio of steel columns comply with code? |  |  |  |  |  |  |
| 21 | Is bay spacing adequate for architectural cladding? |  |  |  |  |  |  |
| 22 | Has expansion joint location been identified on the drawing similar to calculation report? |  |  |  |  |  |  |
| Co-related Documents |  |  |  |  |  |  |  |
| 23 | Has reference been provided for General Notes drawing? |  |  |  |  |  |  |
| 24 | Does drawing include reference of interface drawings (MEP, Civil and architectural drawings)? |  |  |  |  |  |  |
| 25 | Have the referenced drawings, applicable to framing plans been provided? |  |  |  |  |  |  |

| Coordination and Review |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 26 | Has appropriate interdisciplinary and intradepartmental coordination been done? |  |  |  |  |  |  |  |
| 27 | Have reviewers from other disciplines/departments evaluated items pertinent to their area and provided their comments? |  |  |  |  |  |  |  |
| 28 | Has the Responsible engineer resolved their comments? |  |  |  |  |  |  |  |
| 29 | If applicable has the drawing been stamped by a registered professional engineer? |  |  |  |  |  |  |  |
| 30 | Has the drawing been checked for constructability? |  |  |  |  |  |  |  |
| Administrative |  |  |  |  |  |  |  |  |
| 31 | Are Is the Drawing number and the template in accordance with the Project requirements? |  |  |  |  |  |  |  |
| 32 | If revised, have revisions been clearly identified with clouds and clear statement in revision block? |  |  |  |  |  |  |  |
| 33 | Have previous revision indications been removed? |  |  |  |  |  |  |  |
| No. | Checker’s Comments | Resolution |  |  |  |  |  |  |
| Originator's Name / Signature and Date: |  | Checker's Name / Signature and Date: |  |  |  |  |  |  |