| PROJECT NAME: |  |  |  | DRAWING NO. |  |  |  | REV. |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | QUESTIONS |  | ORIGINATOR |  |  |  | CHECKER |  |  |  |
|  |  |  | N/A |  | YES | NO | N/A |  | YES | NO |
| Drawing Presentation |  |  |  |  |  |  |  |  |  |  |
|  | Does the drawing comply with the project CAD Standards (All lines, symbols, legends, abbreviations, text, etc. are legible)? |  |  |  |  |  |  |  |  |  |
|  | Does the legend provide the installation detail regarding the cable rout marker, warning tape, etc.? |  |  |  |  |  |  |  |  |  |
|  | Fields in the title block are consistent with the project drawing log/index. |  |  |  |  |  |  |  |  |  |
|  | Key plan and North arrow are provided & the key plan shall have the layout area hatched. |  |  |  |  |  |  |  |  |  |
|  | Match lines are clearly defined. |  |  |  |  |  |  |  |  |  |
|  | Drawing notes are complete & agree with information on the drawings and details. |  |  |  |  |  |  |  |  |  |
|  | Bar scale is shown on the drawing & correct scale is used for all details, plan/elevation & sections. Details Not to Scale are also clearly marked. |  |  |  |  |  |  |  |  |  |
|  | All interdisciplinary comments & comments from previous revisions have been resolved and incorporated. Holds & revisions are correctly marked. |  |  |  |  |  |  |  |  |  |
|  | Limits of existing and new work including future expansions, interface points, Battery limits are clearly defined. |  |  |  |  |  |  |  |  |  |
| Codes/Standards/Project Requirements |  |  |  |  |  |  |  |  |  |  |
|  | Does the system design comply with the applicable Code, International and Saudi standard, local Government Regulation and Project Specification |  |  |  |  |  |  |  |  |  |
|  | Depth of the duct banks are in accordance with the applicable codes and standards/design criteria/project requirements. |  |  |  |  |  |  |  |  |  |
|  | Manholes are designed and sized in accordance with the applicable codes and standards. |  |  |  |  |  |  |  |  |  |
|  | Direct burial installation is in accordance with the applicable codes and standards. |  |  |  |  |  |  |  |  |  |
| Reference Information |  |  |  |  |  |  |  |  |  |  |
|  | Check to ensure general notes include reference to applicable Codes, Standards and Project Specifications/Supplier submittals. |  |  |  |  |  |  |  |  |  |
|  | Check for correctness of reference drawings. |  |  |  |  |  |  |  |  |  |
|  | List and verify any special requirements by others (Sub-Contractors, Vendors, etc.) |  |  |  |  |  |  |  |  |  |
|  | Sections and details are correctly cross-referenced. |  |  |  |  |  |  |  |  |  |
|  | Conduit stub-up locations are shown with the proper installation detail referenced. |  |  |  |  |  |  |  |  |  |
| Design |  |  |  |  |  |  |  |  |  |  |
|  | Does the layout provide the details of the conduit/raceway type, size and the route |  |  |  |  |  |  |  |  |  |
|  | Does the layout illustrate the underground heat loss and pulling calculation? |  |  |  |  |  |  |  |  |  |
|  | Have the manhole locations coordinated with plant coordinate showing the center line of the manhole? |  |  |  |  |  |  |  |  |  |
|  | Are the manholes located outside hazardous areas and a note has been added to the drawing indicating that the manhole shall be back-filled with clean sand prior to commissioning? |  |  |  |  |  |  |  |  |  |
|  | Does the manhole designed for pulling the cables by ensuring the pulling irons and sufficient hardware to support the cable? |  |  |  |  |  |  |  |  |  |
|  | Are the manholes sized to accommodate the cable racking around sides of the manhole, splicing and cable bend radii? |  |  |  |  |  |  |  |  |  |
|  | Does the layout indicate the ladders inside the manhole, where required? |  |  |  |  |  |  |  |  |  |
|  | Are the distance between the manholes consistent with the maximum tension that may be placed on the cables used, per calculations. |  |  |  |  |  |  |  |  |  |
|  | Have the manhole constructed/ fabricated with provisions for grounding? |  |  |  |  |  |  |  |  |  |
|  | Have the end bells shown for conduit terminations in manholes? |  |  |  |  |  |  |  |  |  |
|  | Are the sump pit provided at manholes, if required? |  |  |  |  |  |  |  |  |  |
|  | Have the provisions made for the duct banks to interface with the manholes so that duct banks will not be affected by differential settlement or heaving? |  |  |  |  |  |  |  |  |  |
|  | Does the conduit with curves, or bends installed in duct banks meet applicable codes and standards requirement for cable pulling and are consistent with the maximum tension and side-wall pressure that may be placed on the cables used, per calculations? |  |  |  |  |  |  |  |  |  |
|  | Does the bending radius in duct bank made with metal fittings to prevent damage to plastic conduit due to pulling tensions? |  |  |  |  |  |  |  |  |  |
|  | Are the installation details appropriate and quantities checked? |  |  |  |  |  |  |  |  |  |
|  | Are the dimensions (width) of duct banks shown on the drawing? |  |  |  |  |  |  |  |  |  |
|  | Are Duct bank sections verified and provide all necessary dimensions and ID chart showing conduit number and assigned service level? |  |  |  |  |  |  |  |  |  |
|  | Do the duct bank sections show proper conduit spacing and do not violate minimum spacing distances? |  |  |  |  |  |  |  |  |  |
|  | Have the duct bank layout checked for interferences? |  |  |  |  |  |  |  |  |  |
|  | Have the duct banks reinforced where crossing drive way or heavy traffic roads? |  |  |  |  |  |  |  |  |  |
|  | Have the consideration been given to water level. If applicable? |  |  |  |  |  |  |  |  |  |
|  | Are the conduit stub-ups shown with proper coordinates? |  |  |  |  |  |  |  |  |  |
|  | Have the steel requirements coordinated with the Civil/Structural? |  |  |  |  |  |  |  |  |  |
|  | Are the selection of the cables appropriate for direct burial and verified the cable suitable for the installation? |  |  |  |  |  |  |  |  |  |
|  | For direct burial, an identifying means and/or protection is installed over the cable in accordance with the design criteria. |  |  |  |  |  |  |  |  |  |
|  | Does the Suitable conduit/cable used in direct burial applications? |  |  |  |  |  |  |  |  |  |
|  | Does the layout provide the installation and sectional details for the directly buried cables |  |  |  |  |  |  |  |  |  |
|  | Does the layout provide the requirement of the cable warning tape and cable markers |  |  |  |  |  |  |  |  |  |
|  | Does the conduit alignment appropriate within the duct bank which enter or connect to the manholes (to avoid and prevent from cable twisting)? |  |  |  |  |  |  |  |  |  |
|  | Does the layout consider the provision or spare raceways for the future? |  |  |  |  |  |  |  |  |  |
|  | Are the layout provided with the proper section cut orientation to avoid cable misrouting? |  |  |  |  |  |  |  |  |  |
|  | Are the layout coordinated with the other external services and ensure the availability or provided with 3D model be utilized to design and coordinate with all underground facilitates? |  |  |  |  |  |  |  |  |  |
| Others |  |  |  |  |  |  |  |  |  |  |
|  | The details should reflect the fabrication/ constructional aspects for the particular item/location. |  |  |  |  |  |  |  |  |  |
|  | Drawing has been verified against the cable schedules. |  |  |  |  |  |  |  |  |  |
|  | Material take off sheets are verified against the IFC issue of the drawing. |  |  |  |  |  |  |  |  |  |
| No. | Reviewer's Comments (against each SLD) | Resolution |  |  |  |  |  |  |  |  |
| Originator's Name / Signature and Date: |  | Checker's Name / Signature and Date: |  |  |  |  |  |  |  |  |