Expro Projects White Book

Volume 6, Chapter 7

Coastal Marine Design Criteria- Template

Document No. EPM-KER-TP-000005 Rev 002

Document Submittal History:

| Rev.: | Rev Status | Date: | Entity | Prepared by: | Reviewed by: | Approved by: |
| --- | --- | --- | --- | --- | --- | --- |
| 00X | For XXXXX | DD/MM/YYYY | XXXXX | NAME | NAME | NAME |
| Signatures: |  |  |  |  |  |  |

THIS NOTICE MUST ACCOMPANY EVERY COPY OF THIS DOCUMENT

IMPORTANT NOTICE

This document, (“Document”) is the exclusive property of Government Expenditure & Projects Efficiency Authority.

This Document should be read in its entirety including the terms of this Important Notice. The government entities may disclose this Document or extracts of this Document to their respective consultants and/or contractors, provided that such disclosure includes this Important Notice.

Any use or reliance on this Document, or extracts thereof, by any party, including government entities and their respective consultants and/or contractors, is at that third party’s sole risk and responsibility. Government Expenditure and Projects Efficiency Authority, to the maximum extent permitted by law, disclaim all liability (including for losses or damages of whatsoever nature claimed on whatsoever basis including negligence or otherwise) to any third party howsoever arising with respect to or in connection with the use of this Document including any liability caused by negligent acts or omissions.

This Document and its contents are valid only for the conditions reported in it and as of the date of this Document.

Table of Contents

1.0	GENERAL	6

1.1	Scope	6

1.2	Codes, Standards and Regulations	6

1.3	Hierarchy of Specifications	6

1.4	Design Life	7

1.5	Units and Datums	7

1.6	Abbreviations	8

1.7	Ship Data	8

1.8	Approach Channel	8

1.9	Navigation Aids	9

1.10	Geotechnical Considerations	9

1.11	Berthing	9

1.12	Mooring	9

1.13	Loading Considerations	10

1.14	Materials	11

1.14.1	Concrete	11

1.14.2	Structural Steel	11

1.14.3	Reinforcement Steel	12

1.14.4	Geotextile	12

1.14.5	Fill Material	12

1.14.6	Rock Armor	13

2.0	NATURAL CONDITIONS	13

2.1	Tide and Water Level	13

2.2	Datums	14

2.3	Bathymetry	14

2.4	Wave and Currents	14

2.5	Wind	14

3.0	WHARVES/JETTIES,QUA WALLS,BREAK WATERS AND REVETMENTS	15

3.1	Wharves/Jetties	15

3.2	Quay Walls	16

3.3	Breakwater & Revetments	16

4.0	LAND RECLAMATION	17

4.1	Site Investigation	17

4.2	Reclamation Fill Level & Fill Material	18

4.3	Reclamation Method	19

4.4	Stability	19

4.5	Settlement	19

5.0	DREDGING	20

5.1	Environmental Considerations	20

5.2	Surveying	20

6.0	SHORELINE PROTECTION	21

6.1	Design Waves	21

6.2	Design Water Levels	21

6.3	Wave Run-up	21

6.4	Wave Overtopping	22

6.5	Height of Protection	22

6.6	Armor Unit Stability	22

6.7	Filters	23

7.0	FOUNDATIONS AND EARTH RETAINING STRUCTURES	23

8.0	PROTECTION SYSTEM/COATINGS	24

9.0	DURABILITY	25

# GENERAL

This section describes the basic format of design criteria which should be included in all types of Costal & Marine Projects.

## Scope

This document covers general design criteria for coastal and marine structures.

## Codes, Standards and Regulations

List the applicable:

National codes,

International codes/standards,

Regulations,

Guideline specifications and

Fire protection codes

[ Any deviations from the codes and regulations to be permitted only after obtaining written approval of the relevant regulatory authorities.]

## Hierarchy of Specifications

[State hierarchy for the project specifications. Hierarchy will be used where a conflict or difference occurs between specification sources.]

## Design Life

Describe required life of:

Quay wall

Jetty

Trestle

Concrete buildings/structures

Steel buildings/structures

Bridges/underpasses/Road culverts

Concrete platforms

Steel platforms/walkways

Retaining walls

Steel/GRP gratings etc.

[State the design life of each structure which is in the scope of work. The design life denotes the duration that the facility will continue to be in service. Mention that structures and elements of structures shall be designed in such a way to ensure a safe and stable behavior during the established design life under the expected loads up to the related national and international standards]

## Units and Datums

Describe:

Unit system

Dimensions

Vertical and Horizontal Datums

## Abbreviations

[Define all abbreviations which will be used in the design criteria.]

## Ship Data

Describe all design vessels and for each vessel specify:

Overall length of Vessel between perpendiculars,

Beam width of vessel,

Laden & unladen drafts,

Loaded displacement, and

Vessel windage area.

## Approach Channel

Identify the required depth of channels based on following parameters:

the loaded draft of the design vessel,

tidal variations, waves, vessel squat and trim,

under keel clearance requirements,

turning basin, and

Sedimentation and maintenance depth.

The required width of channels (width at the dredged level) shall take into account

the beam speed and maneuverability of the design vessel,

passing criteria,

channel depth,

alignment, stability of channel banks,

winds, waves and currents or cross currents and

availability of navigational aids.

## Navigation Aids

[Aids to navigation are used to mark limits of structures such as piers, seawalls, breakwaters and dolphins, channel entrances, boundaries and turns, and hidden dangers such as shoals and rock outcrops, to act as a guide for vessels and to assist with their safe movement.]

## Geotechnical Considerations

[Geotechnical investigations in accordance with project specifications. Specify additional requirements if required.]

## Berthing

Specify the:

Approach velocity of vessel perpendicular to the berthing line

Type of berthing [side berthing, quarter point, third point or mid-point]

Berthing condition [Easy & Difficult Berthing- for sheltered and exposed conditions]

Seawater density

Tug boats assistance in berthing

Type of fender system

## Mooring

Specify the:

Type of mooring system [quick release hook or bollards etc.]

Mooring loads

Mooring lines arrangement

Vertical & horizontal angles of mooring lines

## Loading Considerations

Specify the:

Dead loads

Imposed loads

Live loads [ fork lift trucks, crawler cranes, reach stackers etc.]

Mooring loads

Berthing loads

Equipment loads [marine loading arms, gangway tower etc.]

Rail mounted crane loads

Seismic loads

Lateral earth & water pressures

Surcharge behind walls

Hydrostatic & Hydrodynamic loads

Vessel collision loads

Wind, wave and current loads

Ground water table or tidal lags

Buoyancy loads

Thermal loads

Construction loads

## Materials

### Concrete

Specify the:

Concrete grade (28-day compressive strength) for all (in-situ concrete and Precast concrete) structural elements.

Coefficient of thermal expansion of concrete

Unit weight of mass concrete, in-situ concrete and Precast concrete.

Modulus of elasticity of concrete for short term period

Modulus of elasticity of concrete for long term period

Concrete cover to reinforcement

Concrete exposure conditions

Compressive (28-day) strength of in-situ concrete

Compressive (28-day) strength of precast concrete

Compressive (28-day) strength of mass concrete

Compressive strength of non-shrink grouts

Compressive (28-day) strength of blinding concrete

Compressive (28-day) strength of screed

### Structural Steel

Specify the:

Grade/yield strength of all structural rolled steel sections.

Grade/yield strength of all structural rolled hollow steel sections.

Grade/yield strength of all structural cold form steel sections.

Grade/yield strength of all hot dipped galvanized steel sections.

Grade/yield strength of all stainless steel sections

Coefficient of thermal expansion of steel

Strength of welding electrodes

Type and grade of connection bolts

Type & grade of anchor bolts

Grade/ yield strength of base plate

Type and grade/ yield strength of gratings

Type & grade/ yield strength of Chequered plates

### Reinforcement Steel

Specify the:

Grade and yield strength of deformed bars

Grade and yield strength of plain bars.

Splice lengths for tension laps

Splice length compression laps

Type & grade of mesh reinforcement

### Geotextile

Specify the:

Type of geotextile

Geotextile properties

### Fill Material

Specify the:

Type of fill material (quarry run, crushed rock, etc.)

Angle of internal friction

Moist and submerged unit weights of fill materials

### Rock Armor

Specify the:

Allowable material and properties for armor stones

Allowable material and properties for filter stone

Allowable material and properties for bedding layers

Minimum layer thickness

Requirement for toe/anchor trench

Methods of placement for above and below water application

# NATURAL CONDITIONS

## Tide and Water Level

Specify the following tide levels and the station datum:

Highest Astronomic Tide (HAT)

Mean High Water Level (MHWL)

Mean Low High Water Level (MLHWL)

Mean Water Level (MWL)

Mean High Low Water Level (MHLWL)

Mean Low Water Level (MLWL)

Lowest Astronomic Tide (LAT)

Provide design water level for structures including design return period or annual exceedance probability and the horizon and provision for sea level rise.

## Datums

Specify the project vertical and horizontal datums and unit system

## Bathymetry

Specify the required accuracy, resolution, extend, method of collection and format of bathymetry survey information to be collected and used for design

Provide the:

Reference of bathymetric survey drawings, if applicable.

## Wave and Currents

Specify the:

Operational waves

Extreme waves

Operational currents

Extreme currents

Maximum wave height for moored vessel

Prevailing storm & wave directions

Minimum air gap between highest wave and soffit of beam/slab for design of structures

Design return periods

## Wind

Specify the:

Normal or operational wind speed

Extreme wind speed

Prevailing wind directions

Design return periods

Wind Gust

# WHARVES/JETTIES,QUA WALLS,BREAK WATERS AND REVETMENTS

## Wharves/Jetties

Specify the:

Cope level

Dredged level

Scour protection

Bollard locations

Mooring rings location and elevation

No. of Access ladders and spacing

Fender type and specifications

Equipment details

Crane details, if applicable

Access walkways

Minimum air gap between soffit of beam and highest wave

Provision of inhibit marine growth on piles

Minimum strength of precast concrete at lifting stage

Shear keys for connection between steel pile and concrete plug

Coating/ protection requirement

Crack width limit and concrete covers

Scour protection

Siltation and seabed scour

## Quay Walls

Specify the:

Cope level of capping beam

Founding level

Dredged level

Allowable bearing pressure for surcharge behind wall

Factor of safety in sliding, overturning and eccentricity

Strength of mass concrete precast blocks

Construction tolerances for precast blocks

Scour protection

Quarry run properties

Geotextile properties

Bollard locations

Mooring rings location and elevation

No. of Access ladders and spacing

Fender type and specifications

Specification of rail and distance from cope line, if applicable

Minimum strength of precast concrete at lifting stage

Scour protection

Toe protection

Siltation and seabed scour

## Breakwater & Revetments

[Breakwaters are built to reduce wave action in an area in the lee of the structure. Wave action is reduced through a combination of reflection and dissipation of incoming wave energy.

When used for harbors, breakwaters are constructed to create calm waters for safe mooring and loading operations, handling of ships, and protection of harbor facilities.]

Specify the:

Requirement of crest level determination

Crest surfacing or structure

Fill Material

Filter specifications

Toe protection

Stability factors

Type of Breakwater (fully-, partially- or non-submerged)

Siltation and seabed scour

# LAND RECLAMATION

[The purpose of reclamation is to provide coastal land for roadways, residential development, beach nourishment, port and industrial uses or to improve hydraulic conditions by modifying the coastline.]

## Site Investigation

Specify that:

Site investigation shall be carried out prior to the design of reclamation.

In addition to normal geotechnical investigations required for marine work, the investigations shall cover potential sources of fill materials.

In-situ and laboratory testing of soil samples from within the proposed reclamation area shall be carried out to determine the strength, settlement and permeability characteristics of the underlying soils.

Investigations shall include hydrographic and hydrodynamic study of the currents, waves and sediment transport, environmental impact assessment and marine traffic impact assessment to ensure that there are no unacceptable effects with respect to:

Change in normal and extreme wave

Tidal flushing and water quality

Ecology

Siltation and seabed scour

Shoreline stability of existing beaches

Navigation of large and small vessels

Operation of piers, wharves and cargo-handling areas

Flooding due to tides combined with storm surge, and wave runup

Operation of water intakes that may be affected by reduction of low water levels or sediment buildup.

## Reclamation Fill Level & Fill Material

[Fill sources include borrow material from inland areas and dredged sand. Inland borrow material may be used where it is available, however in general it may not be readily available within short haul distances.]

Requirements for hydraulic fill material obtained from dredging:

Sand shall be free from organic and other deleterious materials. Suitable material shall have less than 10% fines. Lenses of silty or clayey material shall be avoided.

Clayey and silty sand material may be used, but may require significantly more effort for compaction to limit settlement.

Contractor may be required to carry out trial dredging to identify areas where suitable fill material can be obtained.

Material placed immediately behind seawalls, dikes and bulkheads shall be free draining granular material to avoid the unnecessary buildup of water pressure due to tidal lag or ground water flow.

The material and method for placement under water shall be capable of achieving relatively high density fill without compaction.

## Reclamation Method

[Two main reclamation methods are commonly used for reclamation on soft marine seabed layers, the drained method and the dredged method.]

Specify the reclamation methods

## Stability

[The reclamation fill sequence, dredged fill placement level, spacing of vertical drains, magnitude and duration of surcharging are largely determined by stability and settlement criteria.]

Specify the required factor(s) of safety against instability.

## Settlement

Specify the:

Primary consolidation

Secondary Consolidation

Residual Settlement

Estimation of Settlement

Settlement Monitoring method

# DREDGING

The following shall be included in the planning for dredging projects:

## Environmental Considerations

Specific requirements of impact assessment and regulated/acceptable impact levels resulting from dredging and disposal of contaminated material.

Specific acceptable levels of total suspended solids resulting from sediment placement and dredging operations, and describe suitable methods of controlling the operations to ensure environmental protection.

## Surveying

[A variety of hydrographic surveys are to be performed in support of dredging operations, including: preliminary, pre- and post-dredging, acceptance, channel sweep and as-built surveys. Surveys are used to verify the need for deepening the channel, removing obstructions and estimating quantities and cost.]

Specify the:

Type of survey

Surveys for planning or design may be of lower accuracy than pre-dredge or post-dredge surveys.

Type of dredging

[For example, maintenance dredging of navigation channels will require less accuracy than dredging excavation for construction of underwater structure foundations.]

Dredging tolerances

Dredge and survey equipment

[Suction dredges are not as precisely controlled as a resulting survey are of lower accuracy. Mechanical or cutter head dredges which are capable of greater control and can more accurately excavate to the desired bottom profile.]

# SHORELINE PROTECTION

## Design Waves

Specify the:

Design wave height and period which is most critical for structures

The design wave height shall be based on wave modelling, as required due to shore condition

## Design Water Levels

Specify the:

Tides, storm surge and

Rate of future sea level rise or sea level rise for the design life of the structures

## Wave Run-up

[Wave run-up level is important factor affecting the design because it determines the design of crest level of the structures in case no overtopping is acceptable. Wave run-up is the maximum vertical extent of wave uprush on a beach or structure above the still water level (SWL)]

Estimate wave run-up for all shoreline protection structures

Specify the acceptable methods for run-up determination and provide references.

## Wave Overtopping

[Wave overtopping occurs when the run-up level exceeds the structure crest height. Overtopping shall be limited where roads, storage areas, moorings or closed to the structures.]

Specify the:

Critical values of average overtopping discharge

Method of estimation

Overtopping shall be designed for following two conditions:

Operational waves Condition, with no damage

Extreme waves Condition, with some damage to permanent installation

## Height of Protection

The height of protection shall be designed by considering following allowances:

Maximum water level

Anticipated structure settlement

Freeboard

Wave setup

Wave run-up

Overtopping

Sea Level Rise

## Armor Unit Stability

Specify the:

Reference of manual/design guides for armor unit stability.

Types of armor units

Specific weight of the rock, armor units or riprap shall be verified by testing prior to construction.

Installation method shall not cause high impact loads during placing of armor units

Tensile loads during manufacture, shipping and installations, as well as cyclic loads cause by waves, shall not cause cracking and deterioration of armor units.

## Filters

[Filter layers are defined as layers that protect the underlying base material or soil from erosion by waves and currents without excessive build-up of pore pressure in the underlying material. Filters consist of one or more layers of granular material such as gravel or small stone of various grain sizes, geotextile, or a combination of geotextile overlaid with granular material.]

Specify the:

Minimum thickness of granular filter layer

Acceptable gradation

Bedding layers

Geotextile properties

Materials, layer thickness and stability

# FOUNDATIONS AND EARTH RETAINING STRUCTURES

[State coefficient of lateral earth pressures for stability checks, live load surcharge behind the retaining wall and ground water table. Sliding and overturning of foundations should be checked for serviceability limit state. Actual bearing pressure under the foundations shall be less than allowable bearing pressure. Structures shall also be checked for Scouring and erosion protection.

The retaining walls shall be checked for the vehicle collision loads if a road barrier is constructed monolithically on the top of wall.]

State the:

Minimum spacing between piles (driven piles)

Allowable compression, tension and shear for piles

Required factor of safety for sliding, overturning and buoyancy

Required coefficient of friction between soil & concrete

Minimum angle of internal friction of soil

Allowable eccentricity for foundations

Horizontal and vertical service loading on pile, pile group or foundation.

Calculation or reference for sub grade modulus (ks) used in design.

Information and reference about Allowable Bearing Pressure for soil.

Effects of buoyancy and flood should be included in foundation design.

Requirements for engineered fill or other specific placement criteria.

Effects of lateral loading including uplift, overturning and shear transfer to soil

Properties of dry and submerged soil.

High and low groundwater levels including seasonal and long-term fluctuation, and future projection.

# PROTECTION SYSTEM/COATINGS

State the:

Protection system for steel tubular piles and steel structures

Protection system for concrete structures expose to marine environment.

Protection from fire

Cathodic protection system if applicable

# DURABILITY

Specify the Crack width limit for:

Structures with a potential severe exposure condition

Structures subjected to normal exposure condition

Specify the concrete cover requirement for:

Severe exposure conditions

Normal exposure conditions

Fire Protection