1. JOB DESCRIPTION

Project:	Job No. 	 Date:

Lift Description:

Job

Originator: _________________________________________________________________________

2.  LIFT EVALUATION

Based upon the evaluation of alternative methods of access (scaffolding, fixed stairs, ladders, Manlifts, etc.), the use of a suspended personnel platform (Manbasket) is thought to be less hazardous than the alternatives considered.

Job Originator: _________________________________

ES&H Supervisor_______________________________

3.  INSPECTION CHECKLIST

Users must ensure that all items in this checklist are satisfied, including compliance with all safety requirements prior to making a lift.  All precautions and instructions on the decals attached to the crane and the platform must be strictly adhered to.

Crane Requirements

NO	YES

|  |  | All crane inspections are current per requirements, including inspection of the crane hooks. |
| --- | --- | --- |
|  |  | The crane is configured per the completed Personnel Lift Data Sheet, and per manufacturer’s specifications |
|  |  | The crane hook from which the basket will be suspended has a positive locking hook latch. |
|  |  | The correct load chart is with the crane and the operator is thoroughly familiar with all special notes and manufacturer recommendations given on the chart. |
|  |  | All operational aids and safety devices in the crane are functioning and the operator is fully versed in their operation. |
|  |  | The load lines have a 7:1 safety factor (10:1 when using rotation-resistant cable). NOTE: This is achieved by a 50 percent de-rating of the crane load chart. |
|  |  | The crane is on a firm, level surface, the crane is level within 1 percent (1 foot in 100 feet), outriggers are fully extended, down, and locked as applicable. |
|  |  | All load lines are properly reeved, are laying properly on the drums and sheaves, and are free of kinks or other damage. |

Crane Requirements (continued):

NO	YES

|  |  | The boom angle and radius indicators, and the boom length indicators on telescoping cranes are fully functional. |
| --- | --- | --- |
|  |  | The positive acting anti-two block device is functioning properly. A warning system alone does not suffice. |

Rigging Requirements

NO	YES

|  |  | All rigging - slings, shackles, rings, master links, and other rigging hardware - has a minimum safety factor of 5:1. Each component of the rigging system is permanently marked with its safe working load. |
| --- | --- | --- |
|  |  | Each sling or bridle leg is connected to the master link or shackle in a way that ensures the load is evenly distributed between all the bridle legs. For a two or more-leg suspension system, the legs must be designed so that the full load can be supported by just two legs. Synthetic slings are not to be used for personnel lifting. |
|  |  | All wire rope eye fittings are provided with thimbles. |
|  |  | All rigging equipment for the Manbasket is exclusively for that use only. |
|  |  | All rigging has been inspected per company procedure. |
|  |  | Shackles are to be bolt-type with cotter pin. |

Manbasket Requirements

NO	YES

|  |  | The basket has been designed with a 5:1 safety factor by a qualified engineer and welded by a qualified welder. |
| --- | --- | --- |
|  |  | A permanently affixed identification plate is present on the man basket. Required information includes; basket weight capacity and occupancy limit, self-weight, I.D. number, manufacturer. |
|  |  | The guardrail designed to enclose the platform is provided and is enclosed from the toe board to the mid-rail with either solid construction or expanded metal, having openings no greater than 1.3 cm. The toe board must be 10 cm to 15 cm high and the top rail 106 cm high. |
|  |  | A grab rail has been provided inside the entire perimeter |
|  |  | . Body harness anchorage has been provided. |
|  |  | The access gate has been designed to open in only and has a positive locking system to prevent accidental opening during operation. |
|  |  | The design allows enough headroom for employees to stand upright. |
|  |  | There are no rough edges on any man basket surface. |
|  |  | In addition to hard hats, overhead protection is provided when employees are exposed to falling objects. |

4. PRE-LIFT CHECKLIST

NO	YES

|  |  | Precautions have been taken to protect employees from any special hazards in the area where the crane and man basket will be operating; for example, power lines, boom interferences, inadequate ground conditions, or areas where the man basket will be out of the operator’s view. |
| --- | --- | --- |
|  |  | Special precautions have been taken to protect personnel from electrical hazards. When the crane with a man basket is working near electrical lines or devices, the following minimum working clearances shall be observed. |

NORMAL VOLTAGE, KV 						MINIMUM REQUIRED CLEARANCE

(Phase to Phase) 						             	Feet                      Meters

Up to 50 										20                            6.10

Over 50 to 200 									30                            9.15

Over 200 to 350 									40                          12.20

Over 350 to 500 									50                          15.24

Over 500 to 750 									70                          21.34

Over 750 to 1,000 									90                          27.44

NOTE: The clearances listed in the above table are twice the required clearance of that required by the referenced documents

NO	YES

|  |  | Special precautions have been taken to protect personnel from electrical hazards. When the crane with a man basket is working near electrical lines or devices, the following minimum working clearances shall be observed. |
| --- | --- | --- |
|  |  | The operator has performed a thorough inspection of the crane and determined that all systems, controls, and safety devices are activated and functioning properly and that no interferences exist. |
|  |  | The hook is centered over the load. |
|  |  | Travel with the crane is not permitted except where all requirements are satisfied and where not to do so would endanger life. |
|  |  | All load line and boom hoist drums to be operated under full power. Free fall and live boom are not to be used. |
|  |  | The platform must be hoisted in a slow, controlled, cautious manner with no sudden movement of the crane, derrick or platform. |
|  |  | Platform occupants are to perform tasks specified in the Manbasket permit only. Only the number of employees needed for the task at hand are allowed to be hoisted. |
|  |  | Platform occupants are keep all body parts inside the platform during platform movement, with the exception of the signal person. |
|  |  | Platform occupants are not allowed to enter or exit the platform until it is secured to the structure where the work is to be performed, unless securing to the structure creates an unsafe situation. |
|  |  | Platform occupants are not allowed to exit the platform before landing. |
|  |  | Taglines must be used as necessary to control movement of the platform, unless their use would create an unsafe condition. |

4. PRE-LIFT CHECKLIST (continued)

|  |  | The load and boom hoist drum brakes, swing brakes, and locking devices such as pawls or dogs must be engaged when the occupied personnel platform is in a stationary working position. |
| --- | --- | --- |
|  |  | The operator must remain at the controls at all times while the platform is occupied. |
|  |  | Personnel lifts are not to be performed when adverse weather conditions are present, and must be promptly discontinued if there is any indication of dangerous weather conditions or other impending danger. This includes winds in excess of32.2 km/hr., electrical storms, snow, sleet, or conditions that affect clear visibility or the safety of personnel. |
|  |  | The operator must remain in constant contact by standard hand signals or voice communications during operation of crane and man basket |
|  |  | All employees must remain in continuous sight of or in direct communication with the operator or signal person. |
|  |  | The use of a radio is permissible when direct visual contact is not possible, or where the use of a signal per-son could create a greater hazard. |
|  |  | All employees occupying the platform must wear a body harness system, with the lanyard appropriately attached to the lower load block, overhaul ball, or structural member within the personnel platform capable of supporting the fall impact for employees using the anchorage. |
|  |  | All employees occupying the platform must wear a life vest when working over water. |
|  |  | Secure materials and tools to prevent displacement during the lift. |
|  |  | Load the man basket evenly and only carry tools and materials needed for the task at hand. |
|  |  | No other object may be lifted on any of the crane load lines while the platform is suspended. |
|  |  | An audible and visual device has been provided to the personnel in the platform so that they can signal for assistance in the event of an emergency. |
|  |  | Platform occupants are to stand firmly on the floor of the platform, and to not sit or climb on the edge or handrail of the platform or use planks, ladders, or other devices for attaining a work position. |
|  |  | If welding is to be performed by employees occupying the platform; the electrode must be protected from touching the metal components of the platform. |
|  |  | Care must be taken to prevent ropes, electrical cords, and hoses from becoming entangled in the platform when the platform is being moved. |
|  |  | The crane operator responsible for operating the cranes used for personnel handling is a thoroughly trained operator and has related experience operating the subject crane. |
|  |  | All manuals, operating instructions, and load charts provided have been read and understood by the operating personnel prior to starting the operation. |
|  |  | When positioning personnel, the crane operator must ensure that: 1 The platform or boom does not strike any obstruction. 2.The crane boom is not subject to any side load. 3.The platform is not moved until it has been raised a few inches and the brakes applied to ensure that the brakes are functioning properly |
|  |  | The operator and crew have ensured that the area surrounding the platform is clear of personnel and equipment before moving the platform |
|  |  | Prior to hoisting personnel, the man basket has been hoisted a few inches to verify its hang level. |

5. TRIAL LIFT, INSPECTION, PRE-LIFT MEETING SIGN-OFF

All inspections, test and trial lifts – both un-occupied and occupied have been performed, and any deficiencies have been corrected. A pre-lift meeting has been conducted with all personnel involved in the lift, and all employees attending the pre-lift meeting have signed the roster (below).

Meeting Conducted by: ________________________________________________________________

Personnel Lift Supervisor: ______________________________________________________________

Operator: ___________________________________________________________________________

HSSE Representative: _________________________________________________________________

Personnel Lift Authorizer:________________________________________________________________

Crew:

| Name | Name |
| --- | --- |