REV

002

IT LAN-TCP/IP

ETHERNET

SWITCH/ROUTER (TYP.)

GATEWAY (AS REQ.)

EPMS TROUBLE ALARM TO

MAINTENANCE VIA WIRELESS NETWORK

BMS TROUBLE ALARM TO MAINTENANCE

GATEWAY AS REQ. (TYP)

SAC ALARM TO SECURITY

OPT2 TIME SYNC. 
VIA LAN TCP/IP FOR 
IP BASED SYSTEMS

SWITCHBOARDS

MCC, RMU, UPS

SWITCHGEARS

CONTROL/ MONITOR

CONTROL/ MONITOR

CONTROL/ MONITOR

RTU (TYP.)

ELECTRICAL POWER 
MANAGEMENT 
SYSTEM

MONITORING & CONTROL

(REMOTE SWITCHING, POWER,

CONSUMPTION, STATUS ETC.)

MONITORING OF STATUS

BUILDING MANAGEMENT 
SYSTEM (Graphical User 
Interface)

IP CCTV

MONITORING AND CONTROL

SECURITY AND ACCESS CONTROL

UTP CABLE (TYP.)

SAC A ARM TO SECURITY

DOOR

INTERCOM AND ACCESS SYSTEM

MONITORING AND CONTROL

R5 85 (Ms/TP) VIA

MODBUS (TYP)

MASTER CLOCK

SYSTEM	PAVA	IPTV

OPTION 1 TIME SYNCHRONIZATION

FOR NON IP SYSTEMS

SOFTWARE INTEGRATION THRU LAN VIA TCP/IP OR VIA COAXIAL CABLE THRU SET TOP BOXES FOR NON IP TV

Level - 3 - E, External

Electronic documents once printed, are uncontrolled and may become out-dated. Refer to ECMS for current revision

LEGEND:

SOFTWARE INTEGRATION THRU MODBUS VIA LAN TCP/IP

HARDWARE ANALOGUE OR DIGITAL INTEGRATION

DDC	DIRECT DIGITAL CONTROLLER

RTU	REMOTE TERMINAL UNIT

OPTION 1 CONTROL & MONITOR

AUTOMATIC 
TRANSFER 
SWITCH

OPTION 2 - CONTROL

& MONITOR

EMERGENCY 
POWER SYSTEM 
GENERATOR SET

PARALLELING SWITCHGEAR

MONITORING (KVA,

VOLTAGE,PHASE,

ANGLE & FREQUENCY)

LIGHTING CONTROL PANELS

PATIENT ROOM LIGHTING SYSTEM

OPTION 1 - MONITORING OF

CONTINUATION

STATUS

2 WAY 
COMM NICATION 
CA LING

VOIP

NURSE CALL SYSTEM

MONITORING & CONTROL

DDC (TYP.)

PANIC ALARM TO S A &

PATIENT MANAGEMENT TO MS

TIME MANAGEMENT

FROM RTLS

IT LAN TCP/IP

MOBILE PHONES (MAINT.,PATIENT)

PATIENT FILE MANAGEMENT

ADT

WIRELESS NETWORK

RTLS

PATIENT WIRELESS NOTIFICATION

PARKING 
MANAGEMENT 
SYSTEM

MOBILE PHONE (SECURITY)

FDAS

Q-MATIC

R5485 (Ms/TP) VIA

R5485 (Ms/TP) VIA

MODBUS (TYP)

MODBUS (TYP)

000

27/03/19 BORDER CHANGED AND ISSUED FOR USE

24/01/18 ISSUED FOR USE

DA/SY CHAIN FIELD CONNECTION

ENGINEER

REVIEW STATUS

A - REVIEWED; WORK MAY PROCEED

B - REVIEWED WITH COMMENTS; REVISE & RESUBMIT;

C - OBJECTION - REVISE & RESUBMIT; WORK MAY

D - REJECTED

E - REVIEW NOT REQUIRED; WORK MAY PROCEED

WORK MAY PROCEED SUBJECT TO INCORPORATION OF COMMENTS

NOT PROCEED

DATE:

NAME:

REV. No

DATE

REASON FOR ISSUE

DRAWING STATUS

ISSUED FOR USE

KINGDOM OF SAUDI ARABIA	راممقكم راراقم رسمللم

PROJECT REFERENCE -

STANDARD DRAWING

PROJECT TITLE 1

FACILITY No.

XXX

EXTRA LOW VOLTAGE

INTEGRATION BLOCK

DIAGRAM

ARABIC TITLE

PROJECT TITLE 2

PROJECT TITLE 3

ARABIC TITLE

SCALE

CONTRACT No.

DRAWING No.

X

SHEET 1 OF 1

NTS

EPM-KEE-13-000001

002