| ENTITY Logo | ENTITY Name: |  | ENTITY Management Procedure: xxxxxx |
| --- | --- | --- | --- |
| Contract / Lease No. |  |  | Encroachment Permit No: |
|  |  |  | xxxxxx |
| - The Applicant may review ENTITY relevant General Conditions regarding their responsibility for the protection of ENTITY Property and his liability for any actual or possible latent damage done as a result of their negligence in performing work covered by this Encroachment Permit. - A survey should be performed by the Applicant to locate existing marker post, chambers, structures, surface boxes, valves, and other utility appurtenances that shall be affected by any aspect of work authorized under this Permit. Any damage shall be replaced or restored and the area reinstated to its original condition after complete of the work. - Hand excavation / compaction shall be done over and near any utility. Proper protection against mechanical damage must be provided for all exposed pipes. If service interruption is required, a Schedule Utility Outage Request must be submitted to the concerned Utility Group for review / approval prior to proceeding with such work. - ANY DEWATERNG WORK SHALL BE REPORTED TO ENTITY / O&M CONTRACTOR PRIOR TO UNDERTAKING SUCH WORK. - IF ANY EXCAVATION IS DONE AT OR NEAR THE TREATED SANITARY AND INDUSTRY WASTEWATER SYSTEM, SHALL IMMEDIATELY INFORM ENTITY FIELD ENGINEER / O&M CONTRACTOR FOR INSPECTION BEFORE BACK FILLING AND SHOULD OBTAIN A HOLD POINT RELEASE. - There should be no vehicle or construction traffic with heavy axle loads traversing or crossing pipeline. No access or haul road traversing the pipeline right-of-way is permitted. If such crossing is a must, a proper protection shall be submitted for approval - No continued flooding of the pipeline right-of-way shall be permitted. - When the pipe is exposed, it should be adequately supported horizontally to prevent the pipeline from settlement, deflection and subsequently joint leakage. PVC pipe when exposed should be wrapped or covered. - Hold Point Release Number which area attached shall be inspected/witnessed by ENTITY field engineer/O&M Contractor and signed off. - The Construction Contractor shall lay GREEN COLOR marker tape 300mm above each existing utility in the trench during backfilling. It should be of normal ENTITY standards boldly printed word “CAUTION” followed by “IRRIGATION PIPE BELOW.” Upon completion of work, Construction Contractor shall submit a detailed as-built drawing before closing of the Encroachment Permit. - FAILURE OF THE CONSTRUCTION CONTRACTOR TO OBSERVE THESE SPECIAL CONDITIONS COULD RESULT IN A THIRD-PARTY DAMAGE CLAIM FOR ACTUAL AND POSSIBLE LATENT DAMAGE. THE REPAIR COULD INVOLVE EXCAVATION AND EXPOSING ANY UTILITY WITH POSSIBLE LATENT DAMAGE AND ANY IMPLEMENTATION OF ANY NECESSARY CORRECTIVE ACTION. |  |  |  |
| AGREED BY: |  | ISSUED FOR: L/S & IRRIGATION DEPARTMENT (IRRIGATION SECTION) |  |
| SIGN & STAMP: |  | SIGN: |  |
| NAME: |  | NAME: |  |
| DATE: |  | DATE: |  |
| CONTRACTOR: |  | O&M CONTRACTOR'S 24 HOURS SERVICE DESK: |  |
| CONTRACT No. |  | TELEPHONE No. |  |
| TELEPHONE No. |  | [O&M Contractor's 24 Hours Service Desk Hotline Tel No.] |  |