## HSSE Electrical Safety Assessment Checklist

DIRECTIONS: Check either “YES,” “NO,” or “N/A.” If corrective action is required, answer “NO.” For every “NO” answer, provide a brief description of the issue in the “COMMENTS” column. Add the finding / issue to the HSE Tracking Register.

| No. | Electrical Safety | ANSWER |  |  | COMMENTS |
| --- | --- | --- | --- | --- | --- |
|  |  | YES | NO | N/A |  |
|  | Subcategory 1: 120V/240V Single Phase Equipment |  |  |  |  |
| 1 | Do employees visually inspect receptacles, extension cords and equipment connected by cord and plug before each day’s use (to determine whether there are external defects)? |  |  |  |  |
| 2 | Where there is evidence of damage, are the damaged items taken out of service and tagged with a defective tool tag? |  |  |  |  |
| 3 | Is a test for all circuitry performed on all cords and receptacles that are not part of the permanent wiring system? |  |  |  |  |
| 4 | Is all electrical equipment plugged into cords and receptacles equipped with a grounding/earthing line? |  |  |  |  |
| 5 | Are grounding/earthing tests for electrical equipment plugged into cords/receptacles made, recorded and retained by the electrical department or HSE Department? |  |  |  |  |
| 6 | Is all equipment that undergoes the grounding/earthing test marked with electrician’s tape according to the Color Code Schedule? |  |  |  |  |
| 7 | Are all equipment ground/earth conductor continuity tests and electrical continuity and polarity tests performed before first use of the equipment? |  |  |  |  |
| 8 | Are all equipment ground/earth conductor continuity tests and electrical continuity and polarity tests performed at intervals not to exceed 3 months? |  |  |  |  |
| 9 | Does all repaired equipment pass the continuity test before being returned to service? |  |  |  |  |
|  | Subcategory 2: GFCI/ELCB Requirements |  |  |  |  |
| 1 | Where applicable, are ELCBs/RCDs provided for all 120V, single-phase, 15 and 20 Amp receptacle outlets (or 240V, single-phase, 30 and 40 Amp, as applicable) that are not part of the permanent wiring of a building or structure? |  |  |  |  |
| 2 | Are ELCBs/RCDs provided for lavatory, washroom and change room outlets? |  |  |  |  |
| 3 | Are ELCBs/RCDs provided for all areas having a moist or wet atmosphere where electrical equipment or portable electric tools may be used? |  |  |  |  |
| 4 | Are all ELCBs/RCDs inspected and checked as per the quarterly program? |  |  |  |  |
| 5 | Is inspection documentation maintained by the Electrical Department? |  |  |  |  |
|  | Subcategory 3: Power Generators |  |  |  |  |
| 1 | Are generators and welding transformers maintained in good condition? |  |  |  |  |
| 2 | Are fuel tanks located in areas with containment provisions rated to 110% of the total tank contents? |  |  |  |  |
| 3 | Is the area around generators maintained free of oil and diesel spills? |  |  |  |  |
| 4 | Are all rotating components guarded? |  |  |  |  |
| 5 | Is grounding/earthing provided on all mobile electrical generators? |  |  |  |  |
| 6 | Are outlets in good condition with no exposed conductors? |  |  |  |  |
|  | Subcategory 4: Distribution |  |  |  |  |
| 1 | Do distribution boards have ELCBs/RCDs fitted? |  |  |  |  |
| 2 | Are terminal points in an enclosure with feed and outlet cables channeled through the enclosure via rubber/plastic grommets? |  |  |  |  |
| 3 | Does the project prohibit the splicing of cables (i.e., cables will be extended or repaired using the correct fittings)? |  |  |  |  |
| 4 | Are residential standard cables and fittings prohibited onsite (i.e., do all cables meet a construction standard and are resistant to damage/wear and tear)? |  |  |  |  |
| 5 | Is there effective cable management that ensures cables are routed in a manner that does not create an obstruction or trip hazard? |  |  |  |  |
| 6 | Is the method of cable routing in a manner that does not damage or affect the integrity of the cable? |  |  |  |  |
| 7 | Are main isolation points provided in conspicuous and accessible places in workshop/fabrication areas? |  |  |  |  |
| 8 | Are cable connections made with industrial explosion-proof fittings (according to applicable codes and standards)? |  |  |  |  |
| 9 | Do distribution panels carry an Electrical Hazard warning sign? |  |  |  |  |
|  | Subcategory 5: Hand Tools |  |  |  |  |
| 1 | Where 240V hand tools are used, are they double-insulated? |  |  |  |  |
| 2 | Are equipment casings intact with no loose fittings or exposed cables? |  |  |  |  |
| 3 | Are plug fittings of an approved industrial type? |  |  |  |  |
| 4 | Are hand tools in good condition and are tools subject to preventative maintenance schedules? |  |  |  |  |
|  | Subcategory 6: Overhead Transmission Lines |  |  |  |  |
| 1 | Where overhead power transmission lines exist on or adjacent to construction sites or site roads, does the project HSE Execution Plan contain information describing the methods to be used to prevent contact with the overhead lines? |  |  |  |  |
| 2 | Has a risk assessment been conducted to address the overhead lines (e.g., JHA or similar method)? |  |  |  |  |
| 3 | Is work planned, as far as is practical, to avoid close proximity to the overhead lines and accidental contact? |  |  |  |  |
| 4 | Does the supervisor (or electricity provider) specify clearance for given voltages? |  |  |  |  |
| 5 | Where applicable, if work must occur within the specified restriction areas, has a Hazardous Work Permit been obtained? |  |  |  |  |
| 6 | Has the procedure for working around overhead transmission lines been audited and validated as to implementation within 3 months of the project mobilization? |  |  |  |  |