| PROJECT NAME: |  |  | HVAC RISER DIAGRAM NO. |  |  |  |  | REV. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | QUESTIONS |  | ORIGINATOR |  |  | CHECKER |  |  |
|  |  |  | N/A | YES | NO | N/A | YES | NO |
| 1 | Was the correct drawing template used (title block and borders)? |  |  |  |  |  |  |  |
| 2 | Does the title block show the reason for issue/revision, and is the associated revision letter or number appropriate and consistent with Project Procedures? |  |  |  |  |  |  |  |
| 3 | Is the HVAC Riser Diagram number in accordance with Project Procedures? |  |  |  |  |  |  |  |
| 4 | Have all outstanding change papers (e.g., DCNs, FCDs, NCRs, etc.) been incorporated and noted as such in the title block? |  |  |  |  |  |  |  |
| 5 | Is there an Intellectual Property (IP)/Disclaimer Statement on the drawing? |  |  |  |  |  |  |  |
| 6 | Are the symbols for equipment, lines, ducts, valves, dampers, and in-line specialty components in accordance with the Project's standard symbols and legends? |  |  |  |  |  |  |  |
| 7 | Are tag numbers assigned to equipment, lines, ducts, valves, dampers, and in-line specialty components in accordance with Project Procedures? |  |  |  |  |  |  |  |
| 8 | Are line or duct sizes shown on the HVAC Riser Diagram, and are they appropriate for the fluid service? |  |  |  |  |  |  |  |
| 9 | Is there a note on the HVAC Riser Diagram to identify the material and thickness of the pipe or duct shown on the drawing? |  |  |  |  |  |  |  |
| 10 | Are flows and equipment ratings shown on the drawing? (Check "N/A" if there is a note referencing the associated Equipment Schedule with this information.) |  |  |  |  |  |  |  |
| 11 | Do calculations for line or duct sizing support the sizes shown on the drawing? |  |  |  |  |  |  |  |
| 12 | Do fluid velocities fall within industry guidelines? |  |  |  |  |  |  |  |
| 13 | Is there a calculation for the pressure class of the system? |  |  |  |  |  |  |  |
| 14 | Are the pipe or duct materials used rated for the design conditions of the system? |  |  |  |  |  |  |  |
| 15 | Is the direction of flow indicated (if not obvious) for all pipelines or duct runs? |  |  |  |  |  |  |  |
| 16 | Were all the match lines checked on this drawing and found to be correct? |  |  |  |  |  |  |  |
| 17 | Do the line or duct sizes on this HVAC Riser Diagram sheet match other HVAC Riser Diagram sheets at the interfaces (for all off-page connector match points)? |  |  |  |  |  |  |  |
| 18 | Are precise scope boundaries shown on the HVAC Riser Diagram to clearly show the scope of supply for each separate subcontractor and the associated interfaces? |  |  |  |  |  |  |  |
| 19 | Does the system design comply with applicable codes, standards, and regulatory requirements? |  |  |  |  |  |  |  |
| 20 | Does the system design comply with the applicable Project Mechanical Design Criteria, Project Scope Book, or any other Project Design Basis Documents? |  |  |  |  |  |  |  |
| 21 | Are the notes appropriate and are they marked in applicable places on the drawing? |  |  |  |  |  |  |  |
| 22 | Are Standard Details or other drawings referenced in the notes where appropriate? |  |  |  |  |  |  |  |
| 23 | Has appropriate interdisciplinary and intradepartmental coordination of the HVAC Riser Diagram been completed? |  |  |  |  |  |  |  |
| 24 | Has the Originator of the HVAC Riser Diagram resolved coordination comments and incorporated changes where required? |  |  |  |  |  |  |  |
| 25 | On HVAC Riser Diagrams "Issued for Use" (Rev. 000 or higher), have areas of the drawing with incomplete designs or preliminary information been placed on hold? |  |  |  |  |  |  |  |
| 26 | If the HVAC Riser Diagram is being revised, are changes clearly shown (e.g., using clouds, revision triangles, or notes in the revision line of the title block)? |  |  |  |  |  |  |  |
| 27 | If the HVAC Riser Diagram is being revised, are all previous revision markings (clouds or triangles) removed? |  |  |  |  |  |  |  |
| No. | Reviewer's Comments (against each SLD) | Resolution |  |  |  |  |  |  |
| Originator's Name / Signature and Date: |  | Checker's Name / Signature and Date: |  |  |  |  |  |  |