Expro Projects White Book

Landscape Architecture Design Criteria - Template

c

Document No. EPM-KEA-TP-000010 Rev 002

Document Submittal History:

| Rev.: | Rev Status | Date: | Entity | Prepared by: | Reviewed by: | Approved by: |
| --- | --- | --- | --- | --- | --- | --- |
| 00X | For XXXXX | DD/MM/YYYY | XXXXX | NAME | NAME | NAME |
| Signatures: |  |  |  |  |  |  |

THIS NOTICE MUST ACCOMPANY EVERY COPY OF THIS DOCUMENT

IMPORTANT NOTICE

This document, (“Document”) is the exclusive property of Government Expenditure & Projects Efficiency Authority.

This Document should be read in its entirety including the terms of this Important Notice. The government entities may disclose this Document or extracts of this Document to their respective consultants and/or contractors, provided that such disclosure includes this Important Notice.

Any use or reliance on this Document, or extracts thereof, by any party, including government entities and their respective consultants and/or contractors, is at that third party’s sole risk and responsibility. Government Expenditure and Projects Efficiency Authority, to the maximum extent permitted by law, disclaim all liability (including for losses or damages of whatsoever nature claimed on whatsoever basis including negligence or otherwise) to any third party howsoever arising with respect to or in connection with the use of this Document including any liability caused by negligent acts or omissions.

This Document and its contents are valid only for the conditions reported in it and as of the date of this Document.

Table of Contents

1.0	SCOPE	6

2.0	LANDSCAPE DESIGN APPROACH	6

2.1	Contract Requirements	6

2.2	Jurisdictional Boundaries	6

2.3	Applicable Criteria and Standards	7

2.4	Exemptions to Criteria	7

3.0	GENERAL LANDSCAPE DESIGN CRITERIA / DESIGN BASIS REQUIREMENTS	7

3.1	Regulations, Codes, Standards and Guidelines	8

3.2	Protection of Existing Vegetation	9

3.2.1	Guidelines and Requirements	9

3.2.2	Tree Survey	9

3.2.3	Tree Protection Plan	9

3.2.4	Tree Replacement	9

3.3	Plant Materials	10

3.3.1	Basic Plant Types and Definitions	10

3.3.2	Selection of Plant Material	11

3.3.3	Aesthetic Character and Visual Impact	12

3.3.4	Planting Seasons	12

3.3.5	Ground Water	12

3.3.6	Soil Content	12

3.3.7	Erosion and Dust Control, Soil Stabilization	13

3.3.8	Plant installation requirements	13

3.4	Landscape Design Principles	13

3.4.1	General	13

3.4.2	Roadways Driveways and Pedestrian Crossings	14

3.4.3	Sustainability	14

3.5	Landscaping next to buildings	15

3.6	Screening	15

3.6.1	Screening requirements	15

3.6.2	Vehicular Use Area (VUA) Screening	16

3.7	Irrigation Systems	16

3.7.1	Irrigation Design Requirements	17

3.8	Site Furnishings & Elements	17

3.9	Gateways Monuments & Public Art	18

3.9.1	Gateways	18

3.9.2	Monuments	19

3.9.3	Public Art	19

3.10	Plazas Squares & Parks	20

3.11	Hardscape Design	21

3.11.1	Location of Hardscape Material	21

3.11.2	Types of Hardscape Materials	21

3.12	Use of Recycled Materials	22

4.0	ATTACHMENTS	23

Attachment 1 - TREE SETBACK – DISTANCES PROVIDED	24

Attachment 2 - NUMBER OF STREET TREES PROVIDED	25

Attachment 3 - SCHEDULE OF PLANTING	26

Attachment 4 - AVERAGE WATER CONSUMPTION PER SEASON	28

Attachment 5 - IRRIGATION WATER REQUIREMENTS – BY LANDSCAPE MATERIAL	29

Attachment 6 - IRRIGATION WATER REQUIREMENTS – (m3/ha/yr.)	30

Writers Note:

Template Instructions:

Note 1: The text in blue italic is to be used as guidance to the Writer.

Note 2: The text below must be adapted depending on location of the Project, the contractual arrangements, type of project and services provided.

Add project specific criteria here

# SCOPE

Reference the overview of the project provided in the Architectural Design Criteria as defined on Mashroat document EPM-KEA-TP-000003.   Include such items as the major landscape areas, adjacent projects, landscape design packages, imagery, consultants and other general information.

Provide reference of supporting documents wherever required to validate specific information provided in the design criteria.

Add project specific criteria here

# LANDSCAPE DESIGN APPROACH

## Contract Requirements

List the agreements, government environmental laws and scope changes associated with the landscaping for the project.

Add project specific criteria here

## Jurisdictional Boundaries

Identify the Authorities Having Jurisdiction (AHJs) and their responsibilities to issue permits and approvals for landscaping on the project.

Add project specific criteria here

## Applicable Criteria and Standards

All project plans shall comply with applicable local jurisdictional building codes, environmental regulations, associated agency requirements and, Mashroat Project White book Volume 6, Chapter 7, Design Guidelines. Where conflicts or overlapping, criteria exist, the more stringent applies unless otherwise accepted by the local authority.

List any authority design standards and criteria, including manuals, standard drawings, specifications and Building Information Management (BIM) standards.

Add project specific criteria here

## Exemptions to Criteria

Exemptions to authority criteria and standards may be allowed if approved by the project management and authority boards.  The requirements for processing exemptions need to be clarified in the contract and submitted during the review process to be approved.

Add project specific criteria here

# GENERAL LANDSCAPE DESIGN CRITERIA / DESIGN BASIS REQUIREMENTS

## Regulations, Codes, Standards and Guidelines

List all applicable local Saudi Building Codes (SBCs), International codes and standards, regulations and measurement units applicable and considered in the design of project. Provide precedence of codes/ standards, specifications, etc. and advise how discrepancies between codes / standards / specifications shall be addressed.

IBC	International Building Code

ICBN	International Code of Botanical Nomenclature

ICNCP	International Code of Nomenclature for Cultivated plants

MPWH	Ministry of Housing–Kingdom of Saudi Arabia

NFPC	National Fire Protection Code

SBC	Saudi Building Code

ANSI	American National Standards Institute includes American Hort, and American Association of Nurserymen standards

TPI	Turf Producers International

USDA	United States Department of Agriculture Standards

Add project specific criteria here

In case of a conflict between codes, the stricter provisions shall prevail.

## Protection of Existing Vegetation

### Guidelines and Requirements

Root protection zones shall be 1 meter beyond the drip line of the tree. Tree protection fencing shall be a minimum 1 meter high.

Add project specific criteria here

### Tree Survey

A tree survey will be provided to provide better information about the presence and location of significant trees on sites proposed for development.

Add project specific criteria here

### Tree Protection Plan

Depending on the nature of existing site conditions the ENTITY may require a Tree Protection Plan Tree Replacement.

Add project specific criteria here

### Tree Replacement

To satisfy the tree replacement requirements, new trees or existing trees that count for credit must have a combined canopy that will cover a minimum of 10% of the total site area, ten years from the time of development.

See further guidance of Protection of Existing Vegetation in the Landscape Architecture Design Guidelines.

Add project specific criteria here

## Plant Materials

### Basic Plant Types and Definitions

Define the following type and definitions, edit as applicable.

Annuals

Deciduous canopy trees

Evergreen canopy tree

Deciduous understory trees

Evergreen understory trees

Shrubs (deciduous and evergreen)

Grass seed

Groundcovers

Palm

Perennials

Vine

See further guidance of Basic Plant Types and Definitions in the Landscape Architecture Design Guidelines.

Add project specific criteria here

### Selection of Plant Material

The ratio of planting for one gross hectare of land is as follows:

52 trees, 182 shrubs, 70 m2 of groundcover and 294 m2 of grass, and the remaining landscape area is to be covered with gravel mulch 75 mm thick.

The design characteristics or ratio of trees, shrubs, ground cover and grass may be altered if the result satisfies the desires of the ENTITY. An increase in water requirements from such changes must meet the approval of the ENTITY.

Add project specific criteria here

#### General Requirements

The selection of plant materials shall be suitable for the existing and proposed site conditions of the project.

Add project specific criteria here

#### Climate / Environmental Considerations

Plants are to withstand adverse conditions such as airborne pollutants, maximum sun exposure and reflected heat from pavements, high winds and abrasive forces, exposure to salt, road clearing operations and limited root zone soil volumes.

Add project specific criteria here

#### Wind

Wind is a climatic factor that can be either pleasant or unpleasant depending on air temperatures, relative humidity and air velocities. Plants may be used to break, guide, deflect or filter the wind and thereby alter its effects. To properly design for wind control using plant materials, a basic knowledge of air dynamics and information about the directions of prevailing winds and average speeds for different seasons is necessary.

Add project specific criteria here

#### Maintenance Requirements

Groundcovers or shrubs or a combination of both, shall be considered for use in areas that are small and difficult to mow

Add project specific criteria here

### Aesthetic Character and Visual Impact

Plants shall be compatible with building architecture. Colorful, flowering plants shall be used at focal points.

Add project specific criteria here

### Planting Seasons

Unless otherwise specified no planting shall be performed during the months of June, July and August.

Add project specific criteria here

### Ground Water

It is recommended that the saline level of the water used to irrigate or that in the water table on site be tested prior to planting

Add project specific criteria here

### Soil Content

It is recommended that a Geotechnical Report be obtained for all development

Add comments/response here

### Erosion and Dust Control, Soil Stabilization

Within the Community environs all ground areas shall be covered with some material to reduce the problem of blowing sand. In places where the ground is not covered with paving, shrubs, ground cover or grass, it shall be covered with a 75-mm layer of crushed gravel mulch.

Add project specific criteria here

### Plant installation requirements

Plant material shall meet or exceed size and shape relationships specified in the latest edition of the American Standard for Nursery Stock published by the American Association of Nurserymen.

The Entity has the right to certify that landscape plantings have been installed in conformance with the approved landscaping plan including planting sizes, species an, and installation techniques.

Trees shall be planted in accordance to the minimum setbacks as outlined in Appendix A - Tree Setback Distances.

Minimum number of street trees shall be calculated as defined in Appendix B - Minimum Number of Street Trees.

Add project specific criteria here

## Landscape Design Principles

### General

The plants listed in Appendix C shall provide the basis for landscape design. Plant materials and their installation and maintenance shall be in conformance with provisions of the ENTITY’s General Procurement and Construction Specifications (GPCS) and any further amendments.

Reference Tables listed in the Appendices are provided at the end of this Design Criteria to aid designers in selecting plant material. The A/E is required to complete the tables required information:

Add project specific criteria here

Attachment 1	-	Tree Setback – Distances Provided

Attachment 2	-	Number of Street Trees Provided

Attachment 3	-	Schedule of Planting

Attachment 4	-	Irrigation Water Requirements – By Landscape Material

Attachment 5	-	Irrigation Water Requirements – (m3/Ha/Yr.)

### Roadways Driveways and Pedestrian Crossings

Roadways and streets shall be planted.

Driveways: Entry drives in vehicular use areas shall be landscaped with approved trees and shrubs in a manner that visually emphasize the areas and offer the driver and pedestrian an attractive appearance from the street.

Add project specific criteria here

### Sustainability

Landscaping designs shall be designed to balance local environment and require minimal resource inputs. The landscape shall be visually pleasing, cost effective, functional, and maintainable and environmentally sound. Projects are to use principles and guidance by Leadership in Energy and Environmental Design (LEED) by US Green Building Council (USGB) but not required to formally submit for rating.

Add project specific criteria here

## Landscaping next to buildings

The minimum planting distance from building wall is 2 m. For trees with a crown expected to be over 4 m in diameter at maturity, the minimum planting distance from building walls is 1/2 the diameter of the crown, plus 0.5 m.

The minimum planting distance from free standing walls not exceeding 2.5 m in height is 1 m.

Add project specific criteria here

## Screening

Every development shall provide sufficient screening so that neighboring properties are effectively shielded from any adverse impacts of that development or so that the new development is screened from impacts of an adjacent existing use.

It is recognized that because of the wide variety of types of developments and the relationships between them, it is neither possible nor prudent to establish inflexible screening requirements. Therefore, minor deviations may be granted to allow less intensive screening, or requirements for more intensive screening may be imposed, whenever such deviations are more likely to satisfy the intent of the Project.

Add project specific criteria here

### Screening requirements

The minimum required screen landscaping strip shall be 3 m in depth, at minimum, exclusive of utility easements. The screen landscaping strip shall not be located in areas identified for road projects, drainage areas or other public improvement projects.

Berms shall be a minimum of 0.9 m in height, with a maximum of 2:1 slope.

For screening of mechanical equipment and trash collection facilities, the screen heights shall be a minimum of the equipment or facility/enclosure plus 150 mm.

For all other screening purposes, the screen height shall be a minimum of 2.4 meters.

Add project specific criteria here

### Vehicular Use Area (VUA) Screening

Areas used for vehicle service, parking (included structured parking), parking lot drive aisles, and business transactions such as areas adjacent to gasoline pumps (even if under a canopy) and areas for drive-up services, shall be considered Vehicular Use Areas (VUAs). Driveways that are perpendicular or nearly perpendicular to the street and serve as an entrance to the site shall not be considered VUAs for the purposes of this Subsection.

New or renovated parking lots that contain 5 or more parking spaces, or a VUA of 140 square m or more (except for parking associated with single-family or two-family development on single lots).

See further guidance of planting Standards in VUAs in the Landscape Architecture Design Guidelines.

Add project specific criteria here

## Irrigation Systems

The design of irrigation systems shall promote standardization of materials, equipment and hardware to minimize service and maintenance.

In grassed boulevards fronting institutional, commercial and residential uses with potential for heavy pedestrian traffic between the curb and sidewalk, grass areas shall be irrigated. Boulevard trees, shrubs and ground covers shall be watered from an automatic irrigation system.

Landscaped medians in roadways shall be irrigated with subsurface systems.

Urban trees in pavement shall be irrigated with multiple outlet emitter drip irrigation devices.

Add project specific criteria here

### Irrigation Design Requirements

See further guidance of Irrigation Design Requirements in the Landscape Architecture Design Guidelines.

Add project specific criteria here

## Site Furnishings & Elements

Site furnishings shall be compatible with the environment into which they are placed.

Textures, colors, materials, form, and aesthetics of the site furnishing shall compliment the adjacent architecture and/or site.

Maintain and improve the visual quality of the project.

Minimize and standardize maintenance requirements.

Consider the limitations and needs of persons with disabilities to minimize any potential obstructions and hazards.

Minimize life-cycle costs by utilizing high quality products that are durable, backed by a solid warranty and vendor reputations.

See further guidance of Site Furnishings & Elements in the Landscape Architecture Design Guidelines.

Add project specific criteria here

## Gateways Monuments & Public Art

### Gateways

The designs of gateways are subject to the function and width of the road or public space within the right-of-way. To create imaginative and interesting gateways some of the following elements shall be considered:

Vegetation such as trees, shrubs and groundcover plantings shall be used to create visual interest.

Plants used for gateway features shall be hardy, tolerant of the climatic conditions and low maintenance. Irrigation and ongoing maintenance shall be considered when designing planting areas that are associated with gateways;

Sightline requirements for vehicular and pedestrian traffic shall be considered at all gateways.

Pavement treatments can provide a contrast in color and material at gateways.

Materials like masonry, natural stone, or other approved materials may be used construct an architectural component of the gateway.

Hard and soft materials chosen for the gateway shall emphasize the overall design of the gateway and the character of the area where it is located.

Medians that are a part of gateways shall be designed to be accessible as refuges for pedestrians crossing the roadway.

Lighting may be used to create special effects or to accentuate the gateway feature or components of it.

When lighting is used at a gateway, careful attention must be given to the selection and the location of the lights so as not to interfere with the vision of motorists and pedestrians.

Lighting should comply with the general specifications for external lighting including component specifications, energy performance, life expectancy and obtrusive light guidelines.

Signage and graphics may be used to identify the name of the area that the gateway is featuring. Colours and materials of signage and graphics shall not contradict with the gateway or the character of the area where it is located. Graphics and signs if lit must not affect driver vision.

Monuments and Public Art may be used in the design of a gateway.

See further guidance of Gateways in the Landscape Architecture Design Guidelines.

Add project specific criteria here

### Monuments

The monument shall be erected on sites to which the theme of the monument has relevance.

The proposed site shall be the most relevant location available to the monument theme.

The context of the proposed site shall be a major consideration in the mass and scale of the propose object.

See further guidance of Monuments in the Landscape Architecture Design Guidelines.

Add project specific criteria here

### Public Art

The ENTITY will coordinate the site selection of appropriate locations for the installation of public art on publicly-owned space.

The installation/implementation of public art must be conducted in a manner approved by the ENTITY and shall have regard for:

Visibility

Public safety

Interior and exterior circulation

Suitability to location/site (form, content, materials)

Function of existing facility and/or site

Users and their interactions with the public art

Environmental impacts

Reductions of obtrusive light (see ILP GN01 and PLG04).

Public accessibility

Effects on lit environment of adjacent roads

See further guidance of Public Art in the Landscape Architecture Design Guidelines.

Add project specific criteria here

## Plazas Squares & Parks

All Plaza, Squares and Parks shall provide easy and direct access, particularly for the elderly, disabled and young children.

Lighting and Safety shall be a priority, refer to section 5.6 for standards and design guidance documentation.

Lighting should comply with the general specifications for external lighting including component specifications, energy performance, life expectancy and obtrusive light guidelines

See further guidance of Plazas Squares & Parks in the Landscape Architecture Design Guidelines.

Add project specific criteria here

## Hardscape Design

Hardscape is to be used to provide a durable, all-weather surface to accommodate both pedestrian and vehicular travel.

### Location of Hardscape Material

Hardscape is intended generally for the following locations:

Sidewalks and Pedestrian Paths

Plazas and Squares

Transit Stops

Gathering Spaces

Vehicular Use Areas

Add project specific criteria here

### Types of Hardscape Materials

The following list are hardscape materials available for use within ENTITY projects:

Tiles, Bricks and Pavers:

Interlocking Bricks

Cement Tile

Terrazzo Tile

Ceramic Tile

Limestone Tile

Stamped Concrete

Exposed Aggregate Concrete

Cut Stone (Flag Stone)

Clay Stone

Granite Stone

Stone Blocks

Crystal

See further guidance of Hardscape Design in the Landscape Architecture Design Guidelines.

Add project specific criteria here

## Use of Recycled Materials

There are many benefits to incorporating recycled material into a project. Some of them include but are not limited to:

Reduce material and waste disposal costs.

Reduce carbon dioxide emissions

Reduce of environmental impacts

Reduce energy consumption

Reduce project costs

Types of recycled materials that may be incorporated into a construction project include but are not limited to: glass, plasterboard, plastics, wood, aggregates, paper, rubber, compost and other organics, cement replacement.

See further guidance of Use of Recycled Materials in the Landscape Architecture Design Guidelines.

Add project specific criteria here

# ATTACHMENTS

TREE SETBACK - DISTANCES PROVIDED

NUMBER OF STREET TREES PROVIDED

SCHEDULE OF PLANTING

AVERAGE WATER CONSUMPTION PER SEASON

IRRIGATION WATER REQUIREMENTS - BY LANDSCAPE MATERIAL

IRRIGATION WATER REQUIREMENTS - (m3/ha/yr.)

Attachment 1 - TREE SETBACK – DISTANCES PROVIDED

| SETBACK TREES FROM | REQUIRED DISTANCE | PROVIDED DISTANCE |
| --- | --- | --- |
| Underground street light conduit or irrigation main | 0.6 m |  |
| Other underground utilities | 1.2 m |  |
| Street Lighting Poles | 6 m |  |
| Steel utility poles | 3 m |  |
| Driveways | 2 m |  |
| Manholes, valve boxes, services | 1.2 m |  |
| Sewer service boxes | 1.5 m |  |
| Fire hydrants | 2 m |  |
| Road Intersection(s) | 7 m |  |
| Curb face | 0.8 m |  |
| Sidewalk | 0.85 m |  |
| Buildings - fastigiate (columnar) tree | 2 m |  |
| Buildings - regular crown tree | 3 - 5 m |  |

Attachment 2 - NUMBER OF STREET TREES PROVIDED

| Tree Size | Single Family | PROVIDED |
| --- | --- | --- |
| Medium Trees (10 - 20 m ht.) | Greater of 1 per lot or 10 m |  |
| Small Trees (Under 10 m ht.) | Greater of 1 per lot or 7 m |  |

Attachment 3 - SCHEDULE OF PLANTING

| Abbreviation | Botanical Name | Common Name | Arabic Name | Height * (mm) | Caliper * (mm) | Remarks |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TREE SCHEDULE |  |  |  |  |  |  |  |
| PALM TREE SCHEDULE |  |  |  |  |  |  |  |
| SUCCULENTS SCHEDULE |  |  |  |  |  |  |
| GROUND COVER SCHEDULE |  |  |  |  |  |  |
| GRASSES SCHEDULE |  |  |  |  |  |  |

Attachment 4 - AVERAGE WATER CONSUMPTION PER SEASON

|  | Amount (mm/day) | Proposed Consumption (mm/day) |
| --- | --- | --- |
| August (Daily average for entire month) | 1.9. |  |
| October 1 (Equinox) | 1 |  |
| January (Daily average for entire month) | 0.4 |  |

Attachment 5 - IRRIGATION WATER REQUIREMENTS – BY LANDSCAPE MATERIAL

| Landscape Material | Unit | Community Plan Water req/ unit/day | Based on FAO Proj. Water req/ unit/day | Recommended** Water req/Unit/day | Proposed Water req/Unit/day |
| --- | --- | --- | --- | --- | --- |
| Grass | m2 | 42.5 L/m2/d | 11 L/m2/d | 20 L/m2/d |  |
| Shrubs | m2 | 13.4 L/m2/d | * 7 L/m2/d | 11 L/m2/d |  |
| Succulent | * | * | * | * |  |
| Ground cover | m2 | 6.6 L/m2/d | * 4 L/m2/d | 6 L/m2/d |  |
| Trees | No. | 77 L/T/d | 60 L/T/d (3 m dia.) | 90 L/T/d (3 - 4 m dia.) |  |

* The quantity for shrubs and succulent ground cover has been extrapolated from the per meter requirement for trees

** Figures indicate water requirements for mature plants. New plantings will require only a percentage of the above volumes with incremental increase as the plant grows to maturity.

Attachment 6 - IRRIGATION WATER REQUIREMENTS – (m3/ha/yr.)

|  | Extensive* | Intensive** | Proposed |
| --- | --- | --- | --- |
| Yearly Average (Community Plan) | 7,100 | 60,000 |  |
| Yearly Average (Revised) |  | 40,000 |  |
| Maximum (Community Plan) | 9,100 | 75,000 |  |
| Maximum (Revised) |  | 50,000 |  |
| Minimum (Community Plan) | 4,200 | 35,000 |  |
| Minimum (Revised) |  | 25,000 |  |

*Extensive planting (not under drained) 250 trees per hectare planted in sand.

** Intensive planting (under drained) is approximately 20% grass, 50% succulents, 30% shrubs and 50 trees per hectare.