| PROJECT NAME: |  |  | P&ID NO. |  |  |  |  | REV. |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | QUESTIONS |  |  | ORIGINATOR |  |  | CHECKER |  |  |  |
|  |  |  |  | N/A | YES | NO | N/A |  | YES | NO |
| 1 | Was the correct drawing template used (title block and borders)? |  |  |  |  |  |  |  |  |  |
| 2 | Does the title block show the reason for issue/revision, and is the associated revision letter or number appropriate and consistent with Project Procedures? |  |  |  |  |  |  |  |  |  |
| 3 | Is the P&ID drawing number in accordance with Project Procedures? |  |  |  |  |  |  |  |  |  |
| 4 | Have all outstanding change papers (e.g., DCNs, FCDs, NCRs, etc.) been incorporated and noted as such in the title block? |  |  |  |  |  |  |  |  |  |
| 5 | Is there an Intellectual Property (IP)/Disclaimer Statement on the drawing? |  |  |  |  |  |  |  |  |  |
| 6 | Are the symbols for equipment, lines, valves, and in-line specialty components in accordance with the Project's standard symbols and legends? |  |  |  |  |  |  |  |  |  |
| 7 | Are tags assigned to equipment, instruments, lines, valves, and in-line specialty components in accordance with Project Procedures? |  |  |  |  |  |  |  |  |  |
| 8 | Are line sizes, material classes, and ratings (indicating schedule/thickness) shown on the drawing, and are they appropriate for the fluid service? |  |  |  |  |  |  |  |  |  |
| 9 | Do calculations for line sizing support the line sizes shown on the P&ID? |  |  |  |  |  |  |  |  |  |
| 10 | Do fluid velocities fall within industry guidelines? |  |  |  |  |  |  |  |  |  |
| 11 | Is there a calculation for the design pressures and temperatures of the system? |  |  |  |  |  |  |  |  |  |
| 12 | Are the pipe materials used rated for the design conditions of the system? |  |  |  |  |  |  |  |  |  |
| 13 | If a lower rated pipe class is used downstream of a pressure reduction device, is a pressure relief valve or rupture disk required and included in the scope? |  |  |  |  |  |  |  |  |  |
| 14 | Is the direction of flow indicated (if not obvious) for all lines? |  |  |  |  |  |  |  |  |  |
| 15 | Is control logic shown correctly for automatic operation of equipment/system? |  |  |  |  |  |  |  |  |  |
| 16 | Were all the match lines checked on this P&ID sheet and found to be correct? |  |  |  |  |  |  |  |  |  |
| 17 | Do the line sizes and pipe classes on this P&ID sheet match other P&ID sheets at the interfaces (for all off-page connector match points)? |  |  |  |  |  |  |  |  |  |
| 18 | Are precise scope boundaries shown on the P&ID to clearly show the scope of supply for each separate subcontractor and the associated interfaces? |  |  |  |  |  |  |  |  |  |
| 19 | Are flush-out connections shown (if required)? |  |  |  |  |  |  |  |  |  |
| 20 | Does the system design comply with applicable codes, standards, and regulatory requirements? |  |  |  |  |  |  |  |  |  |
| 21 | Does the system design comply with the applicable Project Mechanical Design Criteria, Project Scope Book, or any other Project Design Basis Documents? |  |  |  |  |  |  |  |  |  |
| 22 | Are the notes appropriate and are they marked in applicable places on the P&ID? |  |  |  |  |  |  |  |  |  |
| 23 | Has appropriate interdisciplinary and intradepartmental coordination of the P&ID been completed? |  |  |  |  |  |  |  |  |  |
| 24 | Has the Originator of the P&ID resolved coordination comments and incorporated changes where required? |  |  |  |  |  |  |  |  |  |
| 25 | If required, has the P&ID been stamped by a registered Professional Engineer? |  |  |  |  |  |  |  |  |  |
| 26 | On P&IDs "Issued for Use" (Rev. 000 or higher), have areas of the drawing with incomplete designs or preliminary information been placed on hold? |  |  |  |  |  |  |  |  |  |
| 27 | If the P&ID is being revised, are changes clearly shown (e.g., using clouds, revision triangles, or notes in the revision line of the title block)? |  |  |  |  |  |  |  |  |  |
| 28 | If the P&ID is being revised, are all previous revision markings (clouds or triangles) removed? |  |  |  |  |  |  |  |  |  |
| No. | Reviewer's Comments (against each SLD) | Resolution |  |  |  |  |  |  |  |  |
| Originator's Name / Signature and Date: |  | Checker's Name / Signature and Date: |  |  |  |  |  |  |  |  |