| Activity No. | Activity Description | Inspection/Test Requirements |  | Reference Documentation |  | Method of Verification (see Legend) |  |  | Demonstrated Evidence Report/Checklist Reference No. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Test or Inspection Performed | Stage/ Frequency | Code/Spec/ Etc. | Acceptance Criteria | Construction Contractor | Site Construction Department | Project |  |
| 1.0 | Document review |  |  |  |  |  |  |  |  |
| 1.1 |  | Confirm documents: - Design drawing - Method statement - Material submittal - Mix design - Concrete order form | Prior to commencement | Project Specific | Documents available and latest revision from document management system |  |  |  |  |
| 2.0 | Material receiving |  |  |  |  |  |  |  |  |
| 2.1 |  | Visual inspection for damage, deformity and correct label | Project Specific | Project Specific | Project Specific |  |  |  | Material Receipt |
| 3.0 | Storage |  |  |  |  |  |  |  |  |
| 3.1 |  | Refer to method statement & manufacturer’s recommendation | Project Specific | Project Specific | Project Specific |  |  |  |  |
| 4.0 | Subgrade Preparation, Excavation & Backfill |  |  |  |  |  |  |  |  |
| 4.1 |  | Refer to Earthworks ITP | Project Specific | Project Specific | Project Specific |  |  |  |  |
| 5.0 | Blinding / Mud-mat Line & Grade Check |  |  |  |  |  |  |  |  |
| 5.1 |  | Check & verify the tolerances of levels prior to start fixing reinforcement or formwork. | Pre-pour | Project Specific | Project Specific |  |  |  |  |
| 6.0 | Construction Joint Preparation |  |  |  |  |  |  |  |  |
| 6.1 |  | Check location against drawings | Pre-pour | Project Specific | Project Specific |  |  |  | Pour card |
| 6.2 |  | Contact face – aggregate exposed as per the spec | Pre-pour | Project Specific | Project Specific |  |  |  | Pour card |
| 6.3 |  | All foreign matter and laitance is removed | Pre-pour | Project Specific | Project Specific |  |  |  | Pour card |
| 6.4 |  | Integrity of continuity elements – rebar, water barrier, vapor barrier, etc. | Pre-pour | Project Specific | Element not damaged |  |  |  | Pour card |
| 7.0 | Formwork - general |  |  |  |  |  |  |  |  |
| 7.1 |  | Formwork & false work shall be in accordance with the Project Specific Spec | Pre-pour | Project Specific | Project Specific |  |  |  | Pour card |
| 8.0 | Formwork - Construction |  |  |  |  |  |  |  |  |
| 8.1 |  | Dimensions, line & level | Pre-pour | Formwork drawings | Project Specific |  |  |  | Pour card |
| 8.2 |  | Joint tightness | Pre-pour | Project Specific | Project Specific |  |  |  | Pour card |
| 8.3 |  | All the props, shores, waling, bearers, clamps, wall ties etc., are at the required spacing and adequate. | Pre-pour | Project Specific | Project Specific |  |  |  | Pour card |
| 8.4 |  | Condition of formwork, props, bracing & support | Pre-pour | Project Specific | Project Specific |  |  |  | Pour card |
| 8.5 |  | Check the inserts & box-outs are at correct positions and securely fixed | Pre-pour | Formwork drawings | Project Specific |  |  |  | Pour card |
| 8.6 |  | Check the availability of embedded items to be placed during the concreting and verify their locations | Pre-pour | Rebar drawings | Project Specific |  |  |  | Pour card |
| 8.7 |  | Verify that the edge fillet has been fixed where necessary | Pre-pour | Project Specific | Project Specific |  |  |  | Pour card |
| 8.8 |  | Formwork cleaned and treated as per the spec | Pre-pour | Project Specific | Project Specific |  |  |  | Pour card |
| 9.0 | Reinforcement |  |  |  |  |  |  |  |  |
| 9.1 |  | Reinforcements free from visible flaking or rust, oil, paint, grease, mud or other deleterious matter | Pre-pour | Project Specific | Project Specific |  |  |  | Pour card |
| 9.2 |  | Fixed as shown on the drawings | Pre-pour | Rebar drawings | Project Specific |  |  |  | Pour card |
| 9.3 |  | Main/Secondary rebar according to construction Drawings (numbers/size/ spacing) | Pre-pour | Rebar drawings | Project Specific |  |  |  | Pour card |
| 9.4 |  | Anchorage and lap length | Pre-pour | Rebar drawings | Project Specific |  |  |  | Pour card |
| 9.5 |  | Cover provision | Pre-pour | Project Specific | Project Specific |  |  |  | Pour card |
| 9.6 |  | Links, stirrups, stools & trimming bars according to construction drawings (numbers/sizes/spacing) | Pre-pour | Rebar drawing / schedule | Project Specific |  |  |  | Pour card |
| 9.7 |  | Tolerances on position of reinforcement | Pre-pour | Project Specific | Project Specific |  |  |  | Pour card |
| 10.0 | Embedded Items |  |  |  |  |  |  |  |  |
| 10.1 |  | Anchor bolt material | Pre-pour | Project Specific | Project Specific |  |  |  | Pour card |
| 10.2 |  | Side plate material | Pre-pour | Project Specific | Project Specific |  |  |  | Pour card |
| 10.3 |  | Piping & electrical material | Pre-pour | Project Specific | Project Specific |  |  |  | Pour card |
| 11.0 | Line & Grade Check |  |  |  |  |  |  |  |  |
| 11.1 |  | Verify the line & grade of the formwork, reinforcement, embedment, etc., are according to the drawings & specifications | Pre-pour | Project Specific | Project Specific |  |  |  | Pour card |
| 11.2 |  | Check & verify the verticality of the anchor bolts. | Pre-pour | Project Specific | Project Specific |  |  |  | Pour card |
| 12.0 | Release for Placement |  |  |  |  |  |  |  |  |
| 12.1 |  | Access & working platforms, guard rails and toe boards provision | Pre-pour | Project Specific | Project Specific |  |  |  | Pour card |
| 12.2 |  | Check all the ancillary equipment available such as vibrators, lighting, skips & hand tools | Pre-pour | Project Specific | Project Specific |  |  |  | Pour card |
| 12.3 |  | Pour card sign off | Pre-pour | Project Specific | Project Specific |  |  |  | Pour card |
| 12.4 |  | Verify batching, mixing & delivery of concrete | See Concrete Production ITP | Project Specific | Project Specific |  |  |  |  |
| 12.5 |  | Verify compressive strength | Post pour | Project Specific | Project Specific |  |  |  | Concrete batching ticket |
| 12.6 |  | Sampling as per the spec, check slump & temperature | Post pour | Project Specific | Project Specific |  |  |  |  |
| 12.7 |  | Verify compaction | Post pour | Project Specific | Project Specific |  |  |  |  |
| 12.8 |  | Verify the total elapsed time is within limits | After each concrete load | Project Specific | Project Specific |  |  |  |  |
| 13.0 | Proper Finished Applied |  |  |  |  |  |  |  |  |
| 13.1 |  | Verify the surface finishes and the tolerance on the plumb | Project Specific | Project Specific | Project Specific |  |  |  | Pour card As-built |
| 14.0 | Curing |  |  |  |  |  |  |  |  |
| 14.1 |  | Verify curing procedure, start time and duration | Project Specific | Project Specific | Project Specific |  |  |  |  |
| 14.2 |  | Verify cure temperature | Project Specific | Project Specific | Project Specific |  |  |  | Pour card |
| 14.3 |  | Check for cold joints | Project Specific | Project Specific | Project Specific |  |  |  |  |
| 15.0 | As-Built Survey |  |  |  |  |  |  |  |  |
| 15.1 |  | Verify that the as build survey been done prior to back fill | Project Specific | Project Specific | Project Specific |  |  |  |  |
| 16.0 | Post-pour Testing |  |  |  |  |  |  |  |  |
| 16.1 |  | Compressive strength | Project Specific | Project Specific | Project Specific |  |  |  | Cylinder/cube break test results |
| 16.2 |  | Density | Project Specific | Project Specific | Project Specific |  |  |  |  |
| 16.3 |  | Chloride Penetration | Project Specific | Project Specific | Project Specific |  |  |  |  |
| 16.4 |  | Thermocouple Readings | Project Specific | Project Specific | Project Specific |  |  |  |  |
| 17.0 | Formwork Removed & surface repairs |  |  |  |  |  |  |  |  |
| 17.1 |  | Verify minimum stripping time | Project Specific | Project Specific | Project Specific |  |  |  |  |
| 17.2 |  | Identify surface defects and verify the repair has been done accordance with approved method/specification. | Project Specific | Project Specific | Project Specific |  |  |  |  |
| 18.0 | Final Acceptance |  |  |  |  |  |  |  |  |
| 18.1 |  | All records signed off and closed | Project Specific | Project Specific | Project Specific |  |  |  |  |