## Project Energized Electrical Work Permit (EEWP) Checklist

EEWP Instructions:

A log for EEWPs shall be maintained by the PE, or designee.

EEWP Preparation

- The EEWP Requestor (Foreman, General Foreman, Field Engineer, Superintendent, or Subcontractor Representative) shall fill out Section 1 of the permit.

- The EEWP Requestor shall identify the employees to perform the planned work and ensure they are qualified to perform the work on or near exposed live parts (qualified employees).

- The EEWP Requestor shall sign the permit and obtain the signature of their supervisor and forward the form to the Tagging Authority or designated individual for logging. The Tagging Authority will then provide the form to the RFE.

- The RFE shall ensure the EEWP was logged and shall review the EEWP to determine if the EEWP contains a justification that meets the criteria. Then the RFE signs Section 1 of the permit, and completes Section 2 of the EEWP.

- The RFE enters the permit effective date range. No work authorized by the EEWP shall be performed outside of the permit effective date range.

- The Site Manager or their designee as appropriate, reviews the EEWP and signs Section 3 of the permit to authorize the planned work.

- A pre-job brief is conducted by the RFE once the EEWP is authorized to perform the work. The RFE shall notify the person-in-charge of the work area and work operation covered by the EEWP. Any JHAs / STARRTs that are required shall be developed and/or reviewed by the employees that will perform the planned work.

- Section 4 of the EEWP shall be completed by the RFE as a supplement to the STARRT card. The RFE retains the original EEWP until the permit is closed. The EEWP is kept with the STARRT card at the work location.

- Closed EEWPs shall be returned to the RFE, or designee, for proper filing and logging.

Project Energized Electrical Work Permit (Section 1)

| Permit No. |  |  |  |  |  |  |  |  | Page of |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Section 1 |  |  |  |  |  |  |  |  |  |  |
| Work Package Number: |  |  |  |  | Locator: |  | Component ID Number: |  |  |  |
| Energized Electrical Work Authorization Justification |  |  |  |  |  |  |  |  |  |  |
| De-energization introduces increased hazards. |  |  |  |  |  | De-energization not feasible due to equipment design. |  |  |  |  |
| De-energization introduces additional hazards. |  |  |  |  |  | De-energization not feasible due to operational limitations. |  |  |  |  |
| Justification Explanation: |  |  |  |  |  |  |  |  |  |  |
| Brief Job Description: |  |  |  |  |  |  |  |  |  |  |
| Signatures |  |  |  |  |  |  |  |  |  |  |
| EEWP Requestor (Sign): |  |  |  |  |  |  |  | Date: |  |  |
| EEWP Requestor (Print): |  |  |  |  |  |  |  |  |  |  |
| Responsible Supervisor (Sign) |  |  |  |  |  |  |  | Date: |  |  |
| Responsible Supervisor (Print): |  |  |  |  |  |  |  |  |  |  |
| Concurrence that Proposed Work is Justified |  |  |  |  |  |  |  |  |  |  |
| RFE/SUS (Sign) : |  |  |  |  |  |  |  | Date : |  |  |
| RFE/SUS (Print): |  |  |  |  |  |  |  |  |  |  |
| Qualified Employees |  |  |  |  |  |  |  |  |  |  |
| Person-In-Charge: |  |  |  |  |  |  |  |  |  |  |
| Approach Boundaries: |  |  |  |  |  |  |  |  |  |  |
| Buddy Safety Observer: |  |  |  |  |  |  |  |  |  |  |
| Planned Work: |  |  |  |  |  |  |  |  |  |  |

Project Energized Electrical Work Permit (cont.) (Section 2&3)

| Permit No. |  |  |  |  |  |  | Page of |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Section 2 (Performed by the RFE/SUS) Approach Boundaries to Live Parts for Shock Protection: |  |  |  |  |  |  |  |  |  |  |
| Nominal System Voltage Range, Phase-to-Phase | Limited Approach (Unqualified Person) Boundary |  |  |  |  |  | Restricted Approach Boundary (Qualified Person) |  |  |
|  | Exposed Moveable Conductor |  |  | Exposed Fixed Circuit Part |  |  |  |  |  |
| Class of voltage rated gloves required: |  |  |  |  |  |  |  |  |  |  |
| Flash Hazard Analysis: |  |  | Calculation method |  |  |  |  | Tables method |  |  |
| Detailed Arc Flash Hazard Analysis (Calculation) |  |  |  |  |  |  |  |  |  |  |
| Calculation No. (if performed): Flash Protection Boundary: ft in. |  |  |  |  |  |  |  |  |  |  |
| Incident Energy Exposure to Worker: cal/cm |  |  |  |  |  | Hazard /Risk Category Number: |  |  |  |  |
| Required PPE within Arc-Flash boundary: PPE items will normally be used in conjunction with one another as a system to provide the appropriate level of protection. The Arc Thermal Performance Value (ATPV) for a particular clothing system may be obtained from the AR clothing manufacturer. |  |  |  |  |  |  |  |  |  |  |
| Section 3 - Energized Work Authorization |  |  |  |  |  |  |  |  |  |  |
| Permit effective date range: From: / / To: / / |  |  |  |  |  |  |  |  |  |  |
| RFE/SUS (Sign) : |  |  |  |  |  |  |  |  |
| RFE/SUS (Print) : |  |  |  |  |  |  |  |  |
| Site Manager/Project Startup Manager (Sign): |  |  |  |  |  |  |  |  |
| Site Manager/Project Startup Manager (Print): |  |  |  |  |  |  |  |  |

STARRT Card Supplement Checklist Page (cont.) (Section 4)

|  |  | Section 4 STARRT Card Supplemental Checklist Page of |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Safe Work Requirements: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| YES | N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Pre-job briefing. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Qualified personnel assigned to perform work. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Ensure personnel are available on-site that are trained in cardiopulmonary resuscitation (CPR), and First Aid. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | B/SO with appropriate PPE and trained in the methods of release of individuals from contact with exposed energized parts. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Emergency equipment available for emergency response. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Communications available and in operating condition. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Work area has adequate lighting taking into account dark face shields as required. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Control measures employed to restrict access of unauthorized persons from the work area. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Barriers/ barricades and safety signs to identify and control approach boundaries. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Adequate working clearance and egress routes available. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Conductive items removed (metal jewelry, buckles, badge holders, etc.). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Discuss plan to de-energize in the event of an emergency. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Discuss plan to reinstate or maintain safe work area for all employees when work scope is complete, and/or work shift ends, and/or work breaks occur, and/or site or localized emergencies occur. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Other: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Protective Clothing and Personal Protective Equipment Requirements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Untreated Natural Fiber |  |  |  | AR Clothing |  |  |  | AR Protective Equipment |  |  |  |  | Other Protective Equipment |  |  |  |  |  |  |  |
| YES | NO |  |  | YES | NO |  |  | YES | NO |  |  |  | YES |  | NO |  |  |  |  |  |
|  |  | Short sleeve shirt |  |  |  | Long sleeve shirt |  |  |  |  | Flash suit jacket (multi-layer) |  |  |  |  |  |  |  | Rubber Insulated (V-rated) gloves |  |
|  |  | Long sleeve shirt |  |  |  | Pants |  |  |  |  | Flash suit pants (multi-layer) |  |  |  |  |  |  |  | V-rated tools |  |
|  |  | Pants (Long) |  |  |  | Coverall |  | Head protection |  |  |  |  |  |  |  |  |  |  | V-rated guards |  |
|  |  |  |  |  |  | Jacket, parka, or rainwear |  |  |  |  | Hard hat |  |  |  |  |  |  |  | Insulated hand tools |  |
|  |  |  |  |  |  |  |  |  |  |  | AR hard hat liner |  |  |  |  |  |  |  | Rubber blankets & covers |  |
|  |  |  |  |  |  |  |  | Eye protection |  |  |  |  |  |  |  |  |  |  |  | Testing equipment compatible with voltage present |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Safety glasses |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Safety goggles |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Face and head area protection |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | AR face shield or flash suit hood |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Flash suit hood |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Hearing protection (ear canal inserts) |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Hand protection: |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Leather gloves |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Foot protection: |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Leather work shoes |  |  |  |  |  |  |  |  |  |
| Completed by |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name (print): |  |  |  |  |  |  | (signature): |  |  |  |  |  |  |  |  |  | Date: |  |  |  |
| Section 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Permit Closed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RFE/SUS (print) : |  |  |  |  |  |  | (signature): |  |  |  |  |  |  |  |  |  |  | Date: |  |  |