Expro Projects White Book

Volume 6, Chapter 7

Recycled Water (Irrigation) System Design Criteria Template

Document No. EPM-KEC-TP-000018 Rev 002

Document Submittal History:

| Rev.: | Rev Status | Date: | Entity | Prepared by: | Reviewed by: | Approved by: |
| --- | --- | --- | --- | --- | --- | --- |
| 00X | For XXXXX | DD/MM/YYYY | XXXXX | NAME | NAME | NAME |
| Signatures: |  |  |  |  |  |  |

THIS NOTICE MUST ACCOMPANY EVERY COPY OF THIS DOCUMENT

IMPORTANT NOTICE

This document, (“Document”) is the exclusive property of Government Expenditure & Projects Efficiency Authority.

This Document should be read in its entirety including the terms of this Important Notice. The government entities may disclose this Document or extracts of this Document to their respective consultants and/or contractors, provided that such disclosure includes this Important Notice.

Any use or reliance on this Document, or extracts thereof, by any party, including government entities and their respective consultants and/or contractors, is at that third party’s sole risk and responsibility. Government Expenditure and Projects Efficiency Authority, to the maximum extent permitted by law, disclaim all liability (including for losses or damages of whatsoever nature claimed on whatsoever basis including negligence or otherwise) to any third party howsoever arising with respect to or in connection with the use of this Document including any liability caused by negligent acts or omissions.

This Document and its contents are valid only for the conditions reported in it and as of the date of this Document.

Table of Contents

1.0	General	6

1.1	Introduction	6

1.2	Applicability	6

1.3	Definitions	6

1.4	Abbreviations	6

1.5	General Requirements	6

1.6	Codes and Standards	7

1.7	Review and Approval	7

2.0	irrigation WATER RequirementS	7

3.0	EFFLUENT QUALITY	7

4.0	RECYCLED WATER (irrigation) TRANSMISSION SYSTEMS	8

4.1	Suction and Discharge Water Storage Tanks	8

4.2	Transmission Pipelines	8

4.3	Pumping Stations	8

5.0	IRRIGATION DISTRIBUTION SYSTEMS	8

5.1	Irrigation Pipelines	9

5.2	Valves	9

5.3	Irrigation Control Systems	9

5.4	Screen Filters	9

5.5	Irrigation Fixtures	10

5.6	Subsurface Drainage	10

5.7	Storage Tanks	10

5.8	Pumping Stations	10

6.0	Hydraulic Modelling	10

6.1	Purpose	10

6.2	Modelling Software	11

6.3	Modelling Scenarios	11

6.4	Hydraulic Model Reporting	11

# General

## Introduction

Indicate entity or entities responsible for developing the design criteria. Designate the organization/organizations responsible for design, construction, operation and maintenance.

## Applicability

Define the applicability of design criteria and any limitations.

## Definitions

Explain the different technical definitions used in the design criteria.

## Abbreviations

List all technical abbreviations applied in the design criteria.

## General Requirements

Describe the international publications of design requirements which can also be followed in conjunction with developing the design criteria.

## Codes and Standards

Recycled water systems must comply with all applicable local, National and International codes, standards, and regulations. These codes, standards, and regulations must be noted in the design criteria. Designer shall provide precedence of codes and standards, and ensure that any discrepancies between codes and standards included in the design criteria are addressed.

## Review and Approval

Specify the organization/organizations responsible for reviewing and approving design reports, plans, and specifications.

# irrigation WATER RequirementS

Irrigation water requirements shall be specified to enable calculation of landscape irrigation based on different landscape materials such as grass, shrubs, ground cover and trees. Irrigation water requirements shall be take into account application losses, leaching requirements, soil classification, groundwater table levels, and climate considerations.

# EFFLUENT QUALITY

Design criteria shall specify the required effluent quality requirements that meet the irrigation and landscaping requirements.

# RECYCLED WATER (irrigation) TRANSMISSION SYSTEMS

This section shall encompass the design requirements for irrigation water transmission systems which transport water from wastewater treatment plants to discharge water storage tanks.

## Suction and Discharge Water Storage Tanks

Design requirements shall be described for suction and discharge irrigation water storage tanks.

## Transmission Pipelines

The design requirements shall be described for transmission pipelines, and shall include pipeline slopes, minimum diameter, velocity, pressure requirements, acceptable pipe materials, pipe bedding, different valve locations, thrust blocks, marker posts, and road/service crossings.

Requirements for surge analysis of transmission pipelines, and applicable surge protection structures shall be specified.

There are three different types of air valves, namely air release valves, air/vacuum valves and combined air valves. Design requirements shall include location and sizing of these different valves in the transmission pipeline.

The design requirements for washout valves, including location and sizing, shall be specified.

## Pumping Stations

Design criteria shall be specified for pumping stations required for irrigation transmission pipelines.

# IRRIGATION DISTRIBUTION SYSTEMS

## Irrigation Pipelines

The design requirements shall be described for irrigation distribution pipelines, and shall include pipeline slopes, minimum diameter, velocity, equations to be used for pipe hydraulics, pipe pressures, acceptable pipe materials, pipe bedding, different valve locations, thrust blocks, longitudinal bending limits, joint deflection limits, marker posts, burial depth, and types of pipe joint. Requirements for driplines or lateral lines from solenoid valve boxes to irrigation fixtures shall be specified.

Irrigation techniques for different landscaping types shall be specified, as applicable. The irrigation technique could be a drip emitter system, a bubbler irrigation system, a sprinkler irrigation system, or a subsurface irrigation system.

Technical requirements for irrigation scheduling, and maximum irrigation duration for different types of irrigation techniques shall be specified.

## Valves

General design requirements for valve locations and valve boxes/chambers shall be specified. Different types of valves used in irrigation distribution networks are gate valves, butterfly valves, bubbler solenoid valves, drip solenoid valves, sprinkler solenoid valves, washout valves, air valves, ball valves, pressure reducing valves, pressure sustaining valves, quick coupling valves, and flow control valves.

## Irrigation Control Systems

Design requirements for irrigation control systems and associated electrical components shall be specified.

## Screen Filters

Design requirements for irrigation filters for different types of irrigation systems shall be specified.

## Irrigation Fixtures

Design criteria for different types of irrigation fixtures based on different irrigation techniques shall be specified. The fixtures could be bubblers, sprinklers, emitters, SIS (sandwiched in-line systems), and porous membrane hoses.

## Subsurface Drainage

Design requirements for subsurface drainage shall be specified.

## Storage Tanks

Design requirements shall be described for irrigation water storage tanks required for water distribution networks including configuration, inlet arrangement, outlet arrangement, overflow, washout, access, ventilation, and water level indication.

## Pumping Stations

Design criteria shall be specified for pumping stations required for irrigation distribution pipelines including inlet arrangement, inlet control, pump selection, and outlet control.

# Hydraulic Modelling

## Purpose

Requirements of hydraulic modelling is used as a design tool to evaluate new development and modifications to existing systems, shall be specified.

## Modelling Software

Modelling software shall be specified along with the modelling platform i.e., standalone, CAD integrated or GIS integrated.

## Modelling Scenarios

Modelling scenarios shall be specified in the design criteria so that, on the basis of these scenarios, pressures and velocities in the network can be analyzed.

Modelling techniques shall be described in the criteria; these techniques include steady state analysis, extended period simulation, and reliability analysis.

## Hydraulic Model Reporting

The hydraulic model reporting format shall be specified; this format shall be used when submitting a hydraulic modelling report.