Rigging Work Operations Checklist

|  | RIGGING WORK OPERATIONS Required N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RIGGING HARDWARE/SPECIALTY RIGGING PRE-LIFT CHECKLIST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Will rigging be used today? Yes/No |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Yes | N/A |  |  |  |  |  |  |  |  |  | Yes | N/A |  |  |  |  |  |  |  |  |  | Yes | N/A |
|  | RIGGING INSPECTION CHECK |  |  |  |  |  |  |  |  |  |  | WIRE ROPE SLINGS (All used today) |  |  |  |  |  |  |  |  |  |  | "CHAINFALLS AND COME-A-LONGS" |  |  |  |  |  |  |  |  |  |
|  | Rigging Hardware will be inspected before use? |  |  |  |  |  |  |  |  |  |  | Does the slings have identification tag? |  |  |  |  |  |  |  |  |  |  | Is it clearly marked with manufacturer name, model, serial number, "Safe Working Load Limit"? |  |  |  |  |  |  |  |  |  |
|  | Rigging Hardware Quarterly Inspected: |  |  |  |  |  |  |  |  |  |  | Is diameter of item to lift > the diameter of rope? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | First Quarter - Red |  |  |  |  |  |  |  |  |  |  | Does the Rigger understand the D/d Ratio? |  |  |  |  |  |  |  |  |  |  | Verified it has an annual inspection? |  |  |  |  |  |  |  |  |  |
|  | Second Quarter - Green |  |  |  |  |  |  |  |  |  |  | No more than 3 broken wires in 1 strand/1 lay? |  |  |  |  |  |  |  |  |  |  | Performed the daily "before use inspection? |  |  |  |  |  |  |  |  |  |
|  | Third Quarter - Blue |  |  |  |  |  |  |  |  |  |  | No more than 6 random broken wires in 1 lay? |  |  |  |  |  |  |  |  |  |  | Will the chainfall be used to "pull a load"? |  |  |  |  |  |  |  |  |  |
|  | Forth Quarter - Yellow |  |  |  |  |  |  |  |  |  |  | Checked for severe kinks, abrasion or scrapes? |  |  |  |  |  |  |  |  |  |  | Will the come-a-long be used to lift personnel? |  |  |  |  |  |  |  |  |  |
|  | Foreman/General Foreman VERIFIED! |  |  |  |  |  |  |  |  |  |  | Is Rope over stretched, deformed or corroded? |  |  |  |  |  |  |  |  |  |  | No more than one person will operate the chainfall or come-a-long at any time? |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Instructed all users that there shall be no more than 2 eyes of sling in a single hook? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Yes | N/A |  |  |  |  |  |  |  |  |  |  |  |  | Will the area beneath the chainfall/come-a-long be barricaded to prevent people from walking below a suspended load? |  |  |  |  |  |  |  |  |  |
|  | ROUNDSLING & SYNTHETIC SLINGS |  |  |  |  |  |  |  |  |  |  | Foreman/General Foreman VERIFIED! |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Does the slings have identification tag? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Is the rated for the load that is being picked? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes | N/A |  | The chainfall or come-a-long will be attend at all times? Attended means access no farther than 26' horizontal! |  |  |  |  |  |  |  |  |  |
|  | Does the Rigger understand the D/d Ratio? |  |  |  |  |  |  |  |  |  |  | WIRE ROPE CLIPS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Inspect for tears, cuts, holes, excessive wear? |  |  |  |  |  |  |  |  |  |  | U-bolt clips did not "saddle a dead horse"? |  |  |  |  |  |  |  |  |  |  | The chainfall or come-a-long will be removed and stored properly at end of shift? |  |  |  |  |  |  |  |  |  |
|  | Is sling discolored? Ultraviolet/Sunlight damage? |  |  |  |  |  |  |  |  |  |  | is the correct number of clips being used? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Sling soiled and/or sour/musty smell? |  |  |  |  |  |  |  |  |  |  | Have the clip bolts been torque with a torque wrench to the proper torque value? |  |  |  |  |  |  |  |  |  |  | If No, How long will it remain in service/place? |  |  |  |  |  |  |  |  |  |
|  | Is sling protected from sharp objects or edges? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Will sling be constantly exposed to weather? |  |  |  |  |  |  |  |  |  |  | Have the properly torque clip bolts been clearly marked (orange paint) and identified as verified? |  |  |  |  |  |  |  |  |  |  | NOTE: IF THE CHAINFALL/COME-A-LONG IS LEFT IN PLACE UNATTENDED THEN A BARRICADE MUST BE ERECTED TO PREVENT PERSONNEL FROM ENTERING THE AREA BENEATH THE LOAD. |  |  |  |  |  |  |  |  |  |
|  | If yes, explain why: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Foreman/General Foreman VERIFIED! |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Foreman/General Foreman VERIFIED! |  |  |  |  |  |  |  |  |  |
|  | Specific to Round slings: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes | N/A |  |  |  |  |  |  |  |  |  |  |  |
|  | Any tears, cuts, wear on outer jacket? |  |  |  |  |  |  |  |  |  |  | SHACKLES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes | N/A |
|  | Any exposed central yarns? |  |  |  |  |  |  |  |  |  |  | Clear raised letter markings/rated capacity on? |  |  |  |  |  |  |  |  |  |  | OTHER SPECIAL RIGGING (List) |  |  |  |  |  |  |  |  |  |
|  | Does sling have a "over load cord"? Exposed? |  |  |  |  |  |  |  |  |  |  | The right type of shackle being used for the job? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | If sling has fiber optics test, does light shine? |  |  |  |  |  |  |  |  |  |  | Shackle properly being pulled? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Was the internal core inspected? |  |  |  |  |  |  |  |  |  |  | Inspect shackles for bent, gouges & deep cuts? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Foreman/Supervisor VERIFIED! |  |  |  |  |  |  |  |  |  |  | Foreman/General Foreman VERIFIED! |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRANE LIFT PRE-LIFT CHECKLIST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Will a crane be used today? Yes/No |  |  |  |  |  |  |  |  |  |
|  |  | Pre-Lift Check |  |  |  |  | Yes | N/A |  | Rigging Check |  |  |  |  | Yes | N/A |  | Crane Set-up Check |  |  |  |  | Yes | N/A |  | Personnel Check |  |  |  |  | Yes | N/A |
|  |  | Crane adequate for JOB? |  |  |  |  |  |  |  | Rigging hardware adequate? |  |  |  |  |  |  |  | Equipment Daily Checklist? |  |  |  |  |  |  |  | Person-in-Charge identified? |  |  |  |  |  |  |
|  |  | No lift >90% chart capacity? |  |  |  |  |  |  |  | Rigging properly assembled? |  |  |  |  |  |  |  | Crane on adequate ground? |  |  |  |  |  |  |  | Signal Person(s)) identified? |  |  |  |  |  |  |
|  |  | Payload weight < 20 ton? |  |  |  |  |  |  |  | Softeners used where needed? |  |  |  |  |  |  |  | Crane set-up level? |  |  |  |  |  |  |  | Communication (hand/radio)? |  |  |  |  |  |  |
|  |  | Emergency response needed? |  |  |  |  |  |  |  | Rigging properly tagged? |  |  |  |  |  |  |  | Adequate parts of line? |  |  |  |  |  |  |  | Operator certified/qualified? |  |  |  |  |  |  |
|  |  | Medium Lift Plan? |  |  |  |  |  |  |  | Sling angles > 45 |  |  |  |  |  |  |  | Proper Matting set-up? |  |  |  |  |  |  |  | Non-essential persons out? |  |  |  |  |  |  |
|  |  | Heavy/Critical Lift Plan? |  |  |  |  |  |  |  | Lift point inspect/adequate? |  |  |  |  |  |  |  | Obstructions/Clearance? |  |  |  |  |  |  |  | Rigger roles identified? |  |  |  |  |  |  |
|  |  | Lift Plan signed/approvals? |  |  |  |  |  |  |  | D/d Ratio adequate? |  |  |  |  |  |  |  | Weather Conditions Good? |  |  |  |  |  |  |  | Other craft roles identified? |  |  |  |  |  |  |
|  |  | Pre-Lift Checklist complete? |  |  |  |  |  |  |  | Tag lines length OK to control? |  |  |  |  |  |  |  | Lift area barricaded? |  |  |  |  |  |  |  | Rigging Engineer present? |  |  |  |  |  |  |