| SAFE WORK INSTRUCTIONS |  |
| --- | --- |
| Proposed Actions | Details |
| Description of Task Give brief description of activity (Mopping, Buffing etc.) | Buffing |
| Description of Location Name of contract, building and floor level |  |
| Manager/Supervisor in Charge Person in charge of project with overall responsibility | Qualified Department trainer or Certified Trainer with BICSc License is recommended |
| Minimum competence level of Operative Minimum Training requirements of operative (date of training and by whom) | No assumptions are made that the candidate has any prior experience. Trainer to check before undertaking any form of training |
| Pre Site Preparation Does work area need any associated safety documentation or special procedures before task is undertaken | Where applicable, COSHH data, PAT certificates and any special instruction should be obtained where the trainer is unfamiliar with equipment and materials |
| Equipment required | Appropriate rotary floor machine, Drive disc & pad or brush Mop sweeper and suction cleaner with hose or suction unit fitted to appropriate rotary floor machine (Personal Protective Equipment) PPE Warning signs Circuit breaker Measuring apparatus Cloth |
| Method of work Step by step task SAMPLE procedure | - Wash hands, dry (remove finger rings jewelry, to avoid damage of gloves) and put on appropriate gloves - Put on personal protective equipment - Plan work area - Place warning signs, ensuring all signs are visible - Ventilate the area, if appropriate - Assemble equipment and check for safety - Remove any movable furniture - Dust control the area (with appropriate Mop sweeping procedure) - Damp mop if necessary (Single solution mopping) ensuring the floor is dry before spray cleaning - Attach the floor pad to the machine (refer to manufacturers' instructions) - Unwind cable and plug into a mains socket and a circuit breaker if appropriate - Adjust handle to a suitable/comfortable height making sure arms are straight - Ensure the machine is held firmly in contact with the body and the cable is behind you - Buff area adjacent to skirting by 'cutting in' parallel to skirting - Buff the floor area by moving the machine in continuous, small side to side movements and from front to walking backwards, - On completion or if the floor pad is dirty, remove the floor pad and continue with the remaining area - Remove the plug from the mains socket and re-wind the electricity cable - When the area is completely dry, return the furniture to original position - After use, all equipment should be checked, cleaned, dried and returned to the storage area - Close ventilation if appropriate - Remove warning signs and wipe - Remove PPE and wash hands. |
| Control of Operations Has client officer been notified of work | Trainer should not entre site without informing client Client to indicate area where training and assessments can take place. |
| Storage/Materials/Handling/Cleaning Any special requirements while undertaking task | Work area should be clean and free from any visible debris that can affect the training or the outcome of a task undertaken by the candidate Pad should be washed and air dried Machine should be cleaned from top to bottom, including cable and stored upright in cupboard Do not attempt to pick up machine unless assisted be another trained person |
| Basic safety/Risk/COSHH/Manual Handling Assessments | PAT on all electrical equipment. Do not use if unfit for purpose Where appropriate it should be use as part of the assessment Trainers or candidates should avoid lifting heavy items |
| (Enter corresponding number ref if applicable) | Check the power supply cables and plug for damage, report and label if necessary, remove and do not use Vacuum should be emptied after task Ensure hands are dry when plugging or unplugging appliances When plugging a machine into an electric socket, make sure the switch is in the off position Always make sure to plug the machine into a free plug socket. If none are available, ask a line manager or client before unplugging any other equipment Do not adjust or change the fittings on the machine when it is plugged in Do not place cable over shoulder at any time Before starting, carefully check the internal dust bag is not full and does not contain potentially dangerous items, for example sharps. If it is full, replace it with a new dust bag and check the internal filters are clean and free from debris Do not make any running repairs to machine unless trained and authorized to do so b com an of manufacturer. |
| 3rd. Party Safety controls Measures taken to protect 3rd parties | Cordon off area if possible. Machine should not be left, plugged in if unattended Warning signs should be use as instructed in assessment kit. |
| Personal Protective Equipment | All candidates should be given access to gloves. Appropriate footwear should be advised before candidates undertake training, assessment or both Overalls or equivalent should be worn wear indicated in assessment kit or by instructor. |
| Emergency procedures Additional procedures that may exist due to change in task procedure | Where appropriate refer to COSHH data sheets. Trainer should make themselves familiar of emergency routes and procedure on unfamiliar sites. All candidate should be instructed on the same procedure |
| First Aid requirements | First aid box should be located on unfamiliar sites. Where chemicals are involved instructor to refer to COSHH data sheets |
| Environmental Issues Control of harmful emissions or waste disposal measures | N/A |
| Prepared by, position in company: |  |
| Supervisors name, signed & date: |  |
| Operative name, signed & date: |  |
| Approved by position: |  |