A Sample of Summer Checks

| 1 | Enclosure system |
| --- | --- |
|  | Cleaning of exterior windows, skylights, railing glass, canopies, and other similar surfaces |
|  | Power-washing of pavers on decks and pavers |
|  | Check alignment of exterior doors |
|  | Scheduling of exterior painting projects |
|  | Check operation of sunshades |
| 2 | Electrical systems |
|  | Check operations of photocells and timers for exterior lights on summer daylight hours |
|  | Check and service the operation of cooling equipment in electrical vault |
| 3 | Mechanical systems |
|  | Lift machine room temperature control |
|  | Purging of closed loop mechanical systems |
|  | Replacing of filters in air handling units |
|  | Servicing of air conditioning equipment |
|  | Shutdown of any seasonal heating equipment |
|  | Adjustment of thermostats to summer set points |
|  | Check operations of exhaust fans and supply fans |
|  | Cleaning of dryer exhaust vents |
| 4 | Site work system |
|  | Start-up of irrigation sprinkler system |
|  | Check operation of mowers, pressure washers, ladders and other summer maintenance equipment |
|  | Pressure-washing of walkways and retaining walls |
|  | Cleaning and repair of outdoor furniture |
|  | Implementation of weed control measures |
|  | Roads, platforms and footways, cleaning and gritting |
|  | Storage of temporary winter runners and mats |
|  | Check stability of retaining walls |
|  | Check levelling on roadways and walkways |

A Sample of Winter Checks

| 1 | Mechanical systems - heating & ventilation and chillers |
| --- | --- |
|  | Service central heating equipment |
|  | Check requirements of heat pumps, air handlers and other heating systems for freeze protection and pre-winter season preparation procedures |
|  | Replace/clean filters on equipment, as required |
|  | Check boilers |
|  | Inspect flue pipes and seams for proper support, any holes and cracks |
|  | Clean chimneys and flues as required |
|  | Vacuum vents and terminal/packaged heating devices, as may be required |
|  | Clear any obstacles from heating vents so air can flow freely |
|  | Test operation of HVAC controls and instrumentation such as thermostats and humidistats |
|  | Check for any carbon monoxide leaks |
|  | Check that air intake vents are not blocked |
|  | Check that there is adequate clearance around heating equipment and terminal devices |
|  | Check operation of dampers on exterior walls, such as those at rooftop mechanical rooms |
| 2 | Plumbing & drainage |
|  | Know the location of all catch basins, trench drains and clean outs. Place flags to help identify locations that may become concealed in heavy snowfalls |
|  | Check operation of sump pumps |
|  | Shut off outdoor hose bibs, as required |
|  | Drain garden hoses, store correctly |
|  | Check operation of heat trace cables on exposed pipework |
|  | Drain water fountains and isolate circulation pumps |
|  | Check integrity of insulation on p-traps in exposed parking garages |
| 3 | Fire safety systems |
|  | Check operation of any heat tracing cables on fire sprinkler lines |
|  | Check weatherproofing on any fire detection and fire alarm devices that are mounted on exterior walls or may otherwise be exposed to changing weather conditions |
| 4 | Electrical systems |
|  | Check operation of heat tracing cable and deicing equipment on exposed piping |
|  | Check operation of any electrical heating systems, as required |
|  | Check fuel level for emergency generator and top-up as required |
|  | Check operation of engine block heater on emergency generator |
|  | Check emergency generator cooling system include adequate anti-freeze for local service conditions |
|  | Check exposed wiring and electrical components |
|  | Turn off breakers for outdoor outlets and equipment that is not required during the winter |
|  | Check for any corrosion or other damage on exterior conduits, junction boxes, and panels |
|  | Check operation of all photocells and/or timers to ensure proper lighting control for reduced daylight hours |
| 5 | Lifts and escalators |
|  | Check for any leakage into the elevator or lift pit |
|  | Check that all parts are working as expected |
| 6 | External infrastructure – roadways, walkways, stairs and bridges |
|  | Check hard landscaping surfaces for damage (such as crack, potholes, and differential settlement) that may present a trip and fall hazard or deteriorate the sub-grade. Carry out repairs as required |
|  | Check operation of snow clearing equipment and/or ensure that arrangements are made for snow clearing services |
|  | Arrange for adequate stock on site of snow clearing supplies, such as shovels and ice melt |
|  | Check that stair handrails and guardrails are properly fastened |
|  | Check that stair tread markings are legible and intact |
|  | Clear debris from trench drains, catch basins, sumps, and other collection points |
|  | Demarcate drains for easy identification during heavy snowfalls |
|  | Check that catch basin covers and trench drain covers are properly fastened |
|  | Stake driveways to mark for snowplows (if required) |
|  | Check operation of any ramp heating systems |
| 7 | Landscaping, retaining walls and parks |
|  | Prune tree limbs and foliage that may interfere with roof drains, perimeter drains, and lines of sight to exterior signage |
|  | Apply winter fertilizer to lawns and winter landscape protection, as required (check with landscape contractor for advice) |
|  | Blow compressed air through irrigation sprinkler lines to purge them of water |
| 8 | HTHW, MTHW LTHW |
|  | Check pipe works to ensure no leaks |
|  | Check pumps and ensure that all valves are working to the optimum level |
|  | Check tanks to ensure no leaks or corrosion |
| 9 | Housekeeping & Miscellaneous |
|  | Install appropriate mats/runners at perimeter doorways to minimize transfer of snow ice to interior floor surfaces and to protect against slip and fall hazards |
|  | Cover outdoor furniture or store correctly |
|  | Check that outdoor signage is legible and will not be obstructed by snow accumulations or other winter conditions |
|  | Check with service contractors to confirm their availability and support during the winter months |
|  | Prepare list of emergency contact numbers and provide this to staff and stakeholders |
|  | Replenish stock of winter supplies |
|  | Storage of summer furniture |
|  | Purchase of ice melt supplies and snow clearing |
|  | Roads and walkways maintenance grit management |

A Sample of Autumn Checks

|  | Autumnal Checks |
| --- | --- |
|  | Damage structures, bridges and roads |
|  | Leaves causing blockages to drain |
|  | Leaves cleaning up and clearing in car parks/walkways/pavements/roads etc. |
|  | Roof damages due to high winds and rain |
|  | Drainage blockages and repairs due leaves |
|  | Prune tree limbs and foliage that may interfere with roof drains, perimeter drains and lines of sight to exterior signage |
|  | Lawn and landscape protection and maintenance |
|  | Check roofs structures and walls |
|  | Maintenance of vegetation and weed maintenance |
|  | Check fencing, wall damage, and repair |

A Sample of Spring Checks

|  | Spring Checks |
| --- | --- |
|  | Check drains and sewers etc. |
|  | Check and repair road surfaces |
|  | Check and repair bridges and associated components affected |
|  | Check operation of sump pumps and associated equipment |
|  | Check means of water ingress/penetration into buildings and infrastructure |
|  | Check for water pipes |
|  | Check and to mitigation in place for flooding in critical rooms and areas |
|  | Interior of building water damage mitigations |
|  | Check and repair roof and gutters for leaks |
|  | Check and repair electrical outdoor cables |