Expro Projects White Book

Volume 6, Chapter 7

Structural Design Criteria - Template

c

Document No. EPM-KES-TP-000009 Rev 002

Document Submittal History:

| Rev.: | Rev Status | Date: | Entity | Prepared by: | Reviewed by: | Approved by: |
| --- | --- | --- | --- | --- | --- | --- |
| 00X | For XXXXX | DD/MM/YYYY | XXXXX | NAME | NAME | NAME |
| Signatures: |  |  |  |  |  |  |

THIS NOTICE MUST ACCOMPANY EVERY COPY OF THIS DOCUMENT

IMPORTANT NOTICE

This document, (“Document”) is the exclusive property of Government Expenditure & Projects Efficiency Authority.

This Document should be read in its entirety including the terms of this Important Notice. The government entities may disclose this Document or extracts of this Document to their respective consultants and/or contractors, provided that such disclosure includes this Important Notice.

Any use or reliance on this Document, or extracts thereof, by any party, including government entities and their respective consultants and/or contractors, is at that third party’s sole risk and responsibility. Government Expenditure and Projects Efficiency Authority, to the maximum extent permitted by law, disclaim all liability (including for losses or damages of whatsoever nature claimed on whatsoever basis including negligence or otherwise) to any third party howsoever arising with respect to or in connection with the use of this Document including any liability caused by negligent acts or omissions.

This Document and its contents are valid only for the conditions reported in it and as of the date of this Document.

Table of Contents

1.0	GENERAL	8

1.1	Scope	8

1.2	Codes, Standards and Regulations	8

1.3	Hierarchy of Specifications	9

1.4	Design Life	9

1.5	Units, Elevations and Datum	10

1.6	Abbreviations	10

2.0	SURVEY AND SETTING OUT	10

3.0	GEOTECHNICAL INFORMATION	11

4.0	GENERAL DESIGN REQUIREMENTS	11

4.1	General	11

4.2	Strength	12

4.3	Serviceability	13

4.4	Durability	14

4.5	Risk Category	14

5.0	DESIGN LOADS	14

5.1	General	15

5.2	Special Considerations	15

5.3	Applicable Loads	15

5.4	Dead Loads	16

5.5	Live Loads	16

5.6	Roof Live Loads	16

5.7	Rain Loads	16

5.8	Flood Loads	17

5.9	Wind Loads	17

5.10	Seismic Loads	18

5.11	Self-Straining Loads	18

5.12	Loads Due to Lateral Earth Pressures, Ground Water	19

5.13	Hoisting Devices Loads	19

5.14	Crane Loads	20

6.0	LOAD COMBINATIONS	20

7.0	MATERIALS	20

7.1	Concrete	21

7.2	Structural Steel	22

7.3	Reinforcement Steel	22

8.0	PROTECTION SYSTEM/COATINGS	23

9.0	FOUNDATION AND EARTH RETAINING STRUCTURES	23

10.0	HIGH RISE BUILDING STRUCTURES	24

10.1	Structural System	24

10.2	Codes and Standards	24

10.3	High Rise Building Limits	25

10.4	Wind Loads	25

10.5	Seismic Loads	25

10.6	Damping System	26

11.0	TRANSPORTATION STRUCTURES	26

11.1	Design Service Life	27

11.2	Codes and Standards	27

11.3	Design Loads	28

11.4	 Loads Combinations	29

11.5	Bearings	29

11.6	Depth/Span Ratios	29

11.7	Expansion Joints	29

11.8	Crash Barrier and Handrail	29

11.9	Deflection and Camber Limits	29

11.10	Durability	30

12.0	NON-BUILDING STRUCTURES	30

12.1	Culverts and Drainage Structures	30

12.2	Manholes and Chambers	31

12.3	Liquid Retaining Structures	32

12.4	Thrust Blocks	32

12.5	Mechanical Equipment Support Structures	33

12.6	Electrical and Communication Structures	33

12.7	Supports for Signs, Luminaries, Traffic Signals	33

# GENERAL

This section describes basic format of structural design criteria template which should be included in all types of structural design basis.

## Scope

The design criteria template covers the following:

Site location

Applicable codes, standards and regulations

Environmental data

Geotechnical data

Specifications

Loads and load combinations

Serviceability limits

Strength and stability limits

## Codes, Standards and Regulations

List the applicable:

National codes,

International codes/standards, including fire code

Regulations, and

Guideline specifications

[Any deviation from the codes and regulation to be permitted after obtaining written approval of the national authorities/Entity.]

## Hierarchy of Specifications

[State hierarchy for the project specifications. Hierarchy will be used where a conflict or difference occurs between specification sources.]

## Design Life

Describe design service life of:

Concrete buildings/structures

Structural Steel buildings/structures

Bridges/underpasses/Road culverts

Concrete platforms

Steel platforms/walkways

Earth and water retaining structures

Structural Steel/GRP gratings etc.

[State the design service life of each structure which is in the scope of work. The design service life denotes the duration that the facility will continue to be in service. Inspection and maintenance program shall be defined. Mention that structures and elements of structures shall be designed in such a way to ensure a safe and structurally sound behavior during the prescribed design service life under the expected loads in accordance to the national and international codes and standards]

## Units, Elevations and Datum

Describe:

Unit system

Dimensions

Elevation/Datum

## Abbreviations

[Define all abbreviations which will be used in the design criteria.]

# SURVEY AND SETTING OUT

Describe the coordinate system and datum for the geographic location of structures.

Provide the reference of topographic survey drawings.

Provide the reference of bathymetric drawings [if applicable]

# GEOTECHNICAL INFORMATION

Provide the reference of geotechnical report which generally includes the following:

A plot showing the location of all test bore borings and/or excavations.

Elevation of the water table, if encountered.

Recommendations for foundation type

- [design criteria, including net allowable bearing pressure and allowable peak bearing pressure of the stratum of soil where shallow footings would normally and provisions to mitigate the effect of expansive and highly plastic soils.]

Recommended type of cement

Electrical resistivity of the soil, and

Protection measures for concrete surfaces in contact with soil.

# GENERAL DESIGN REQUIREMENTS

## General

State that:

The adopted Model Building Codes provides minimum design requirements, however the A/E shall resort to the codes, standards and regulations listed in Section 1.2 for governing design requirements.

The building structures shall be configured to comply with Fire code.

Different types of metals shall be electrically isolated wherever undesired galvanic action may possibly occur.

## Strength

State that:

All structures and elements of structures shall be designed in such a way to ensure a safe and structurally sound behavior during the prescribed design service life under the expected loads in accordance to the national and international codes and standards.

Buildings and all other structures, and all parts thereof, shall be designed and constructed to resist safely the factored loads in the load combinations without exceeding the strength limit states for the material of construction.

All materials shall comply with national/international standards and project specifications, and

The material shall meet the basic properties as required by these criteria (coefficients of thermal expansions, unit weights, etc.).

## Serviceability

[Serviceability limit state is the function of a building/structure type, its appearance, maintainability, durability and comfort of its occupants to be preserved under normal use.]

Describe the camber, deflection, drift and vibration.

Camber

- [Provide allowable values for cambers for each type of element, like simply supported beam, cantilever beam.]

Deflection

- Specify the:

- Vertical deflection limits for all structural elements.

- Horizontal displacement limits for all structural elements.

- Deflection limits for crane runway beams

Drift

- [The drift of structure shall be evaluated under service loads to provide the required serviceability performance of the structure, including integrity of interior partition and exterior cladding. The drift under strength loads combinations shall not cause collision with adjacent structures and also mention the drift for building having overhead gantry cranes.]

Vibration

- [Floor system susceptible to vibration shall be designed so that vibration would have no significant adverse effect on the intended occupancy of the building.]

## Durability

Specify the crack width limit for:

Structures for severe exposure conditions

Structures structure for normal exposure conditions

Specify the concrete cover requirement for:

Concrete cover requirement for severe exposure conditions

Concrete cover requirement for normal exposure conditions

Fire protection

## Risk Category

Building occupancy category

- [Each building and structure shall be classified in accordance with national/international standard. Mention that occupancy or risk category shall not be lower than the occupancy category specified therein.]

State the importance factor for floods, wind and earthquake.

# DESIGN LOADS

## General

[Design loads and their sources shall be clearly indicated on construction documents in accordance with the provisions of Design Basis. Any conflict in definition, notion, terminology, or methodology between Design Basis and the other codes and standards referenced in Design basis shall be evaluated and reconciled. Where there is conflict between standard and other available data, the most current acceptable data or practice shall be used.

Design loads not provided in the Design Basis shall be determined and established in accordance with references and standards listed in section 1.2. Loadings not covered by the references and standards in this Design Basis shall be obtained from available technical literature, manufacturer’s data, or be rationally derived. Such derivations shall be described in the Design Basis report.]

## Special Considerations

[Factors that may result in differences between actual and calculated loads shall be considered in the design]

## Applicable Loads

[All loads which will be used in design, shall be clearly specified including dead loads, live loads, surcharge loads, seismic loads, sand accumulation load on inaccessible roofs, equipment loads, moving loads, crane loads including dynamic allowance, wind loads, lateral earth pressures, accidental loads, vehicle collision loads, mechanical & electrical services loads (e.g. cable trays loads, piping loads, weight of cables, etc.) and temporary construction loads.

Members of the structures shall be designed for the combinations of loads and force effect that can occur simultaneously to produce the most critical design conditions.]

## Dead Loads

[For purpose of design, the actual weights of materials of construction and permanent equipment shall be used. In the absence of definite information, reasonably estimated conservative weight values shall be submitted for review and approval.

Where tanks, vessels, bins, or other containers of significant weight are located in the structure, the dead weight of the item shall be applied to the structure at the points of support. The dead weight reactions of connected pipelines and supported conveyance systems shall also be included in the design.

Minimum allowance shall be added to account for weight of mechanical ducts, electrical cabling, plumbing runs, etc. A reasonable margin will be used when vendor equipment loads are unknown or at preliminary stage. A reasonable allowance shall be added to the self-weight of floor or roof system to account for additional weight of concrete slab due to design development. The additional dead weight of screed on roof and floors for drainage slope shall be considered. Where a roof garden is located on a structure, self-weight of soil (wet) shall be applied to the structure. The weight of equipment’s pedestals or supports on roof shall be considered.]

## Live Loads

[Live loads used in the design of buildings and other structures for use and occupancies which are not specified in the Design Basis shall be based on the maximum loads expected for the intended use or occupancy.]

## Roof Live Loads

State the minimum roof live load for accessible roofs.

State the minimum roof live load for inaccessible roofs.

## Rain Loads

State the rain load and also specify the minimum rain load to be used in the design.

## Flood Loads

State the flood load if applicable.

## Wind Loads

State the following:

Design wind speed

Exposure category

Risk Category

Importance Factor

Extreme wind speed and normal wind speed

[Buildings and other structures, including the main wind force-resisting system and all components and cladding thereof, shall be designed and constructed to resist wind loads as specified code and standards.]

## Seismic Loads

For seismic design specify the:

Risk Category

Importance Factor

Lateral Load Resisting System

Response Reduction Factor (R)

System Over Strength Factor (Ω0)

Deflection Amplification Factor (Cd)

Site Class (Should be referenced to Geotechnical Report)

Site Coefficients

Mapped Spectral Response Accelerations

Spectral Response Coefficients.

[Every building or other structure, and portion thereof, shall be designed and constructed to resist the effects of earthquake motions.]

## Self-Straining Loads

Specify design minimum temperature

Specify design maximum temperature

Specify maximum metal temperature

## Loads Due to Lateral Earth Pressures, Ground Water

Specify the:

Coefficients of lateral earth pressures,

Unit weight of soil (dry and submerged),

Angle of internal friction, coefficient of friction between soil

[Load effect on basement walls and/or underground structures due to lateral earth pressure and ground water shall be calculated in accordance with the codes and standards listed in section 1.2]

## Hoisting Devices Loads

[Structures that support hoisting devices (cranes, monorails, jibs, etc.) shall be designed for the forces induced by the operation or movement of the components of the hoisting device. Hoisting device loads shall be considered live loads. Impact, braking, and other relevant dynamic forces shall be considered in the design. For exterior devices, the effect of wind shall be considered in the design.

For devices that can move freely when exposed to wind, safety restraint mechanisms shall be provided. For hoisting devices that are exposed to the local environment such that dust, and, windborne corrosive chemicals can accumulate on the device, the most robust protective coating system available from the manufacturer shall be used.]

## Crane Loads

Specify the:

Crane Capacity

Clear Span

Height of lift

Clearances (Top, bottom, each side, longitudinal)

Limits of hook travel (vertical, horizontal)

Limits of crane travel (longitudinal)

Wheel spacing

Longitudinal force parallel to crane runway beam

Transverse force perpendicular to crane runway beam and maximum wheel load reaction including dynamic allowance.

Weight of hoist trolley

[All vertical, lateral, and longitudinal crane forces (reactions) shall be transferred through a clearly defined and continuous load path to the ground. All lateral forces shall be transferred through frame action to the transverse bracing system of the building. All longitudinal forces shall be transferred as axial loads through girders and brace members. Brace systems shall preferably be tension-only X-braces. Brace connections shall be designed as slip-critical and configured with minimal eccentricity, and shall transfer forces into rigid elements of the main structural members.]

# LOAD COMBINATIONS

Building and other structures shall be designed using the allowable or ultimate strength provisions of codes as listed in section 1.2.

# MATERIALS

## Concrete

Specify the:

Required, 28-day, compressive strength) for all structural elements.

Coefficient of thermal expansion of concrete

Unit weight of concrete/ mass concrete

Modulus of elasticity of concrete for short term period

Modulus of elasticity of concrete for long term period

Concrete covers

Concrete exposure conditions

Compressive strength of grouts

Required Compressive (28-day) strength of mass concrete

Required Compressive (28-day) strength of precast concrete

Compressive (28-day) strength of blinding concrete

Compressive (28-day) strength of screed

## Structural Steel

Specify the:

Grade/yield strength of all structural rolled steel sections.

Grade/yield strength of all structural rolled hollow steel sections.

Grade/yield strength of all structural cold form steel sections.

Coefficient of thermal expansion of steel

Strength of welding electrodes

Type and grade of structural connection bolts

Type & grade of anchor bolts

Grade/ yield strength of base plate

Type & grade of metal gratings

Type & grade/ yield strength of chequered plates

Grade/yield strength of all hot dipped galvanized steel sections.

## Reinforcement Steel

Specify the:

Grade and yield strength of deformed bars

Grade and yield strength of plain bars.

Splice lengths for tension laps

Splice length compression laps

Type & grade of mesh reinforcement

# PROTECTION SYSTEM/COATINGS

State the waterproofing & fireproofing system

State the protection system or coating system for structural steel sections.

Specify the hot dipped galvanizing system if provided.

State the coating system for underground structures exposed to aggressive soil and ground water.

State the cathodic system if applicable.

# FOUNDATION AND EARTH RETAINING STRUCTURES

[State coefficient of lateral earth pressures for stability checks, live load surcharge behind the retaining wall and ground water table. Sliding and overturning of foundations should be checked for serviceability limit state. Actual bearing pressure under the foundations shall be less than allowable bearing pressure. Calculated foundation settlements should not exceed the allowable settlements given in the geotechnical report.

The retaining walls shall be checked for the vehicle collision loads if road barrier is constructed monolithically on the top of wall.]

State the:

Factor of safety for sliding, overturning and for buoyancy

Coefficient of friction between soil & concrete

Angle of internal friction of soil

Allowable eccentricity for foundations

Horizontal and vertical service loading on pile, pile group or foundation.

Calculation or reference for sub grade modulus (ks) used in design.

Information and reference about Allowable Bearing Pressure for soil.

Effects of buoyancy and flood that should be included in foundation design.

Requirements for engineered fill or other specific placement criteria.

Properties of dry and submerged soil.

Ground water table for buoyancy.

# HIGH RISE BUILDING STRUCTURES

[The scope of this subsection is to provide guidance on design and construction of high rise building structures constructed of concrete and steel materials. The design requirements provided herein, or cited by reference, shall be based on local and intentional industry standards.]

## Structural System

[Complete description of the lateral force resisting systems and of any special mechanisms used for the dampening or control of lateral response in high rise building structure shall be provided in the design basis report.]

## Codes and Standards

List the applicable:

National codes,

International codes/standards,

Regulations,

Guideline specifications and

Fire protection codes

## High Rise Building Limits

Specify the:

Story drift limits with respect to the structural system

Drift limit for wind loads

Drift limit for Seismic loads

## Wind Loads

Specify the:

Design wind speed

Exposure category

Risk Category

Importance Factor

## Seismic Loads

Specify the:

Risk Category

Importance Factor

Vertical and Lateral Force Resisting System

Response Reduction Factor (R)

System Over Strength Factor (Ω0)

Deflection Amplification Factor (Cd)

Site Class (Should be referenced to Geotechnical Report)

Site Coefficients

Mapped Spectral Response Accelerations

Spectral Response Coefficients.

## Damping System

[Dampers assist in the dissipation of energy generated by seismic shaking and wind gust frequency. Passive Dampers are uncontrolled and require no power to work. Active Dampers generate forces on a structure to counter wind or seismic imposed disturbance.]

Specify the:

Type of damper

Location of damper

# TRANSPORTATION STRUCTURES

[Structures shall have adequate configurations acceptable to the Entity and shall meet aesthetic and functional requirements of the project. Structures configurations shall include features such as simplicity, regularity, integrity, redundancy, and ease of inspection, maintenance, and repair. Structures shall demonstrate the high standards for safety, durability, and cost effectiveness.]

## Design Service Life

Specify the:

Bridges

Underpasses

Road culverts

Road Side Crash Barriers

Retaining walls

Approach slabs

Steel platforms

Walkways

Gratings

Overhead Gantry signs

Sign boards

## Codes and Standards

Specify the codes/standards for:

Bridges

Concrete Structures

Steel Structures

Piling

Overhead Gantry signs

Earth retaining structures

Steel connections

Anchor bolts and base plates

Design of Bearings

## Design Loads

Specify the:

Dead load and superimposed dead load

Live load

Impact load

Wind load on structure

Wind Load on live load

Centrifugal force

Longitudinal force from live load

Earth lateral load

Braking force

Longitudinal force due to friction or shear resistance at expansion bearings

Other forces, rib shortening, shrinkage, temperature and/or support settlement

Earthquake forces

[Transportation structures carrying highway loads shall be designed for seismic loads based upon the project requirements and standards. Structures designed as such shall satisfy the specified performance criteria to ensure safety of the public and minimize the costs of damage, and loss of use caused by potential earthquakes.The soil supporting the transportation structures shall be analyzed for liquefaction hazard and, as appropriate, mitigation measures shall be taken.]

Stream flow pressure

- [All piers and other portions of structures which are subject to the force of flowing water or drift shall be designed to resist the maximum stresses induced thereby.]

Buoyancy

- Specify the ground water table

- [Design of foundations is negatively impacted by buoyancy. The foundation structures shall be designed assuming the negative effects of buoyancy, unless water below the foundation level is effectively removed.]

## Loads Combinations

Specify the load combinations including load factors for bridge loadings.

## Bearings

Specify the:

Types of bearings

## Depth/Span Ratios

Specify the depth/span ratios for all types of transportation structures.

## Expansion Joints

Specify the movement limits of expansion joints.

## Crash Barrier and Handrail

Specify the:

Type of crash barrier

Vehicle collision loads on barrier

Hand rail details and specifications

## Deflection and Camber Limits

Specify the deflection and camber limits

## Durability

Specify the:

Crack width limits for substructure

Crack width limits for superstructure

Concrete cover for all project exposure conditions

# NON-BUILDING STRUCTURES

[Structures shall have adequate configurations acceptable to the Entity and shall meet aesthetic and functional requirements of the project. Structures configurations shall include features such as simplicity, regularity, integrity, redundancy, and ease of inspection, maintenance, and repair. Structures shall demonstrate the high standards for safety, durability, and cost effectiveness.]

## Culverts and Drainage Structures

Specify the requirement for:

Backfill material properties

Ground water table

Soil pressure which shall be considered fully saturated unless drainage system installed.

The soil above the culvert shall be considered as a saturated if there is ground water table or surface water.

Water stops in construction & expansion joints

Bedding material

Scour protection

Concrete cover requirements for exposed to water and soil conditions

Lifting arrangement for precast culverts

Slope from upstream to downstream

Minimum stable slopes for excavations

Loading conditions (Empty culvert with full external load, etc)

## Manholes and Chambers

Specify the requirement for:

Backfill material properties

Ground water table

Water stops in construction & expansion joints

Concrete cover requirements for exposed to water and soil conditions

Minimum stable slopes for excavations

Loading conditions (Empty chamber with full external load, etc)

Surcharge loading for permanent and construction stage

Traffic load on top of cover if applicable.

Type of cover (fixed or removable)

Sump pit for collection of drainage water

Elevation and size of openings or recesses

Rungs details (materials, spacing, width)

Allowable factors of safety for buoyancy or uplift.

Allowable bearing pressure and minimum eccentricity.

Lifting lugs in the walls for precast manholes or chamber.

Spreader beams to be used for vertical lifting for very large manholes or thin sections

## Liquid Retaining Structures

Specify the:

Backfill material properties

Ground water table

Surcharge loading for permanent and construction stage

Water stops in construction & expansion joints

Concrete cover requirements for exposed to water and soil conditions

Minimum stable slopes for excavations

Loading conditions (Empty chamber with full external load, etc)

Allowable factors of safety for sliding, overturning and buoyancy.

Allowable bearing pressure and minimum eccentricity.

## Thrust Blocks

Specify the

Location and type (fixed, allow horizontal movement, etc.) of thrust blocks

Minimum soil cover thrust block

Field test pressure for which thrust blocks shall be designed

Ground water table effect on thrust blocks

## Mechanical Equipment Support Structures

Specify the:

Equipment loads and fixing details

Code reference for static loads on foundations

Code reference for dynamic loads on foundations

Isolation of dynamic foundations through the use of isolation or expansion joints from adjacent foundations to allow independent system movement.

Piping reactions for each load condition like wind, operational, transient, seismic and temperature

Types of pipe supports (fixed, roller, hinge, etc.)

## Electrical and Communication Structures

Specify the:

Static, rotating and vibrating loads and the locations.

Rotating and vibrating equipment which shall have isolated foundations

Structural supports of rotating and vibration equipment which shall be designed to avoid resonant vibration with equipment and limit amplitude to acceptable values. The acceptable values shall be verified from vendors.

Openings including its size and elevations

Allowable settlement

## Supports for Signs, Luminaries, Traffic Signals

Specify the:

Relevant codes

Wind speed

Allowance for increase in area exposed to wind which shall be considered in the design.

Geotechnical investigation at location of foundation.

Hot dipped galvanization or protected steel structures which shall be used.