Expro Projects White Book

Template - Escalator Data Sheet

c

Document No. EPM-KEA-TP-000007 Rev 001

Table of Contents

1.0	ESCALATOR TECHNICAL DATA	3

1.1	Escalator - General Data	3

1.2	Design Data	3

1.3	Drive & Control System	3

1.4	Drive Machine	4

1.5	Balustrade	4

1.6	Steps	4

1.7	Truss	5

1.8	Track System	5

1.9	Step Chains	5

1.10	External Interfaces	5

1.11	Control and Operation Features	5

1.11.1	Safety Features	5

1.11.2	Emergency Features	7

1.11.3	General Operation Features	7

1.11.4	Method of Intermittent (standby speed) Operation	7

1.11.5	Certificates	7

Writers Note:

Template Instructions:

Note 1: The text in blue italic is to be used as guidance to the Writer.

Note 2: The text below must be adapted depending on location of the Project, the contractual arrangements, type of project and services provided.

# ESCALATOR TECHNICAL DATA

Construction Contractor will supply Escalators [Product Name and Model Name], manufactured in [Manufacturer’s Location], which complies with Project Specifications section [Reference Specification Number] with the following technical features:

## Escalator - General Data

| Escalator | Technical Features |
| --- | --- |
| Product |  |
| Assembly Factory |  |
| Safety Regulation code | [EN115-1:2008+A1:2010] |
| Carrying Capacity | XXXXX persons/hour (As defined in EN115- 1:2008+A1:2010 for 1000mm step width and 0,5m/s escalators for every manufacturer) |
| Rated speed | 0.X m/s |
| Angle of inclination | XX degrees |
| Floor to floor height | XXXX mm |
| Step Width | XXXX mm |
| Flat steps | X [(4 steps for more than 19m rise as per NFPA requirement)] |
| Duty (Hours per Day) | XX h / X days per week |
| Condition of use | [Indoor / Outdoor] |
| Certificates | [To be released after manufacturing for each unit as per EN 115.] |

## Design Data

| Escalator | Technical Features |
| --- | --- |
| Loads to be used for design and testing structural frame, rails, and power transmission components | Structural Frame: XXXX N/m2 Rails XXXX N/m2 Power Transmission: XXXX N/m2 (breakage load) Live Loads (including loads of intended use) X.XX KN |
| Loads used for design and testing of motor and brakes | Motor: 120 Kg/step Brakes: 120 Kg/step (As per EN115) |
| Loads used handrail for design and testing of handrail. | >=25kN |

## Drive & Control System

| Escalator | Technical Features |
| --- | --- |
| Drive Type | [Input drive type, e.g.AC Motor + Worm Gear + Duplex Chain] |
| Supply power net type | [e.g.3x Phases (Power) + 1x Phase (Lighting)] |
| Main power supply voltage & tolerance | XXX V (Power) / XXX V (Lighting). Tolerance: ±5% |
| Frequency | [ e.g.60Hz] |
| Recommended operating ambient temperature range | XºC to XXºC |
| Maximum operating ambient temperature range | XºC to XXºC |
| Relative Humidity range | 0-XXX% |
| Controller enclosure Insulation rating (IP) | [e. g. IP55 Control Cabinet] |
| Certificates - TM221 | [provide certificate number] |

## Drive Machine

| Escalator | Technical Features |
| --- | --- |
| Motor type | AC Motor |
| Motor Output power | [e. g. 9.5kW, 10,5kW; 12kW; 13,5; 15kW; 16,5kW; 19kW; 2x10,5kW; 2x12kW; 2x13,5kW; 2x15kW; 2x45kW] |
| RPM | [ e. g. 1500rpm (Working with VVVF)] |
| Starting method | [ e.g. Inverter] |
| Motor Noise | [ e.g. XX dBA measured at 1m height from the comb plate] |
| Protection | [ e. g. IP65] |
| Insulation Rating | [ e. g. Class F] |
| Running Current | [ e.g. 21A, 25A, 28A, 32A, 35A, 39A, 45A] |
| Drive Machine type | [ e. g. AC Motor + Worm Gear + Duplex Chain] |
| Brake Type | [ e. g. Double Electromechanical brake on Motor Shaft] |
| Certificates: | [Provide certificate numbers] |

## Balustrade

| Escalator | Technical Features |
| --- | --- |
| Finishing | [ e. g. SLIM line. Clear rectangular toughened safety glass [10mm] thickness. 1000mm height.] |
| Alignment | [ e. g. Glass, inner-outer decking and cladding alignment (Joints perpendicular to the handrail path)] |
| Decking Profiles | [ e. g. Stainless Steel AISI304 Satin Finish N4 [2mm thickness] |
| Handrail Guide Finishing | [ e. g. Stainless Steel] |
| Handrail Finishing | [ e. g. Thermoplastic Urethane Handrail] |
| Skirt Panels Finishing | [ e. g. Stainless steel AISI304 satin finish N4 [3mm thickness]] |
| Skirt LED Lighting | [ e. g. Aluminum profile with white plastic cover] |
| Floor Plates finishing | [ e. g. Natural finish with Black anodized grooves] |
| Comb Plates finishing | [ e. g. Natural finish with Black anodized grooves] |
| Certificates | [Provide as required] |

## Steps

| Escalator | Technical Features |
| --- | --- |
| Material | [ e. g. Aluminum (GD Al Si 12) |
| Construction | [ e. g. Die Cast Aluminum |
| Step Rollers | [ e. g. Polyurethane 75 x 24mm. RS Outdoor Sealing. |
| Certificates | [Provide as required] |

## Truss

| Escalator | Technical Features |
| --- | --- |
| Outer Cladding | [ e. g. Stainless steel AISI304 satin finish N4 [1.5mm thickness] |
| Truss protection | [ e. g. Epoxy paint (90 microns) suitable for outdoor use and drainage at lower head.] |
| Steel thickness | [ e. g. Steel Profiles: S275JR Sand Blasted, open commercial profiles (UPN, LD,) various shapes/thickness (minimum 6mm) Soffit Plate: Oil-tight Steel 4mm thickness] |
| Intermediate support required above: | [ e. g. 6m rise as standard configuration.] |
| Certificates | [Provide as required] |

## Track System

| Escalator | Technical Features |
| --- | --- |
| Track material | [ e. g. Galvanized Steel Sheet] |
| Radio of curvatures | [ e. g. Upper transition: 1500mm; Lower transition: 1050mm] |
| Certificates | [Provide as required] |

## Step Chains

| Escalator | Technical Features |
| --- | --- |
| Chain roller arrangement | [ e. g. Step chain rollers fitted inside chain link plates 135 / 101,15mm Sections] |
| Maximum pin Pressure | <=XXN/mm2 |
| Lubrication method | [ e.g. Automatic Lubrication with pre-level and Alarm] |
| Certificates | [Provide as required] |

## External Interfaces

(Mention the accounted external interfaces with a brief description)

| Escalator | Technical Features |
| --- | --- |
| Escalator Supports | [ e. g. Damping pads are fitted beneath the support stands at both ends of the supporting structure to prevent vibration transfer to the building. The adjustment screws on the stands allow fine set-tings to be made at any time. The stands require a recess measuring 200 x 140 mm prior to installation. Deviations of ± 15 mm can be compensated.] |

## Control and Operation Features

(Expand the sheet & Enter ALL the accounted features under the below main headings with a brief description)

### Safety Features

EN115-1:2008+A1:2010

- All EN115 Safety requirements/features included

Electromechanical ratchet on main shaft (with UPS)

Emergency brake in case of drive chain breakage

Handrail speed sensor with Alarm

Stops the escalator and ring an alarm if the handrail speed is ±15% for more than 2s

Missing step safety device

Stops the escalator if there is a gap in the step band

Open machinery space safety switch

Stops the escalator if the landing plate is removed

Skirting safety brushes

Prevent users resting their shoes against the skirt

Brake function monitoring

Stops the escalator if the brake is not released on starting the unit or the brake fails to be applied on stopping

Pit lighting

Portable Lamp to light the pits

Lock for floor plates

Prevent non-authorized access to the pit

Anti-static electricity roller for handrails

Prevent static electricity in the handrail

Step band lock

To lock the step band during brakes/motor maintenance

Comb plate lighting

Provides light to the coms-step joint

Halogen free wiring

Increase safety in case of fire

Drive chain monitoring

Monitors the status of the drive chain and activates the auxiliary brake

Brake shoe wear safety device

Monitors the status of the brake shoe

Service brake: upwards force to be 1/3 of downwards force

Prevents high decelerations

Service brake

Service brake monitoring contact against failure or phase change

Skirting micro switches at landings [no. of landings]

Stop the escalator if any object gets trapped between the skirt and a step

Failure display at inner decking profile

Display the status of the escalator

Fire relay

To stop the escalator in case of fire

Reverse operation

To reverse direction of operation during station evacuation mode

Metallic trays for the wires

To guide the wires safely

Line-driver differential

Motor thermal protection

Prevents motor overheating

Asymmetry relay/phase sequence

Prevents phase reversals

Escalator speed control

Prevents under-over speeds

Horizontal comb plate safety switch

Stops the escalator if an object gets trapped between the combs and steps

Handrail inlet switch

Prevent trappings between the handrail and handrail inlet (handrail entry guard)

Step chain tension device

Stops the escalator if the step chain beaks or is not tensioned

Stop button in pit

Stop button for the maintenance operator

Step run guard

To stop the escalator in case of step/chain sag - located before upper and lower reversing station. It is positioned so that the step band can be halted before the defected step reaches the comb plate, in order to prevent injury to the users.

Safety signals at landings

Safety indication for the users

Stop button in pit

Stop button for the maintenance operator

Step run guard

To stop the escalator in case of step/chain sag

Safety signals at landings

Safety indication for the users

Low friction skirts

Prevent trappings with the skirts

Braking distance control

Measures the braking distance

Securities reset in cabinet

Reset all the securities after a failure

### Emergency Features

Emergency user stop

- User Stop at upper and lower landing

### General Operation Features

PLC with Ethernet controller suitable for remote control

Reversible: Direction key switch selector at handrail inlet

Stop button at handrail inlet

Running time meter

### Method of Intermittent (standby speed) Operation

Stand by operation by means of an Inverter (0,5m/s - 0,2m/s) and a detection photocell in the skirting panel. Standby speed can be modified under requirement.

### Certificates

[Provide as required]